2-1-3 20-Pin, 24-Bit, 192kHz D/A with Volume Control (WM8772)

PIN CONFIGURATION

PIN DESCRIPTIONS

| PIN | NAME | TYPE | DESCRIPTION |
| --- | --- | --- | --- |
| 1 | MODE | Digital input | Control format selection 0=Software control 1=Hardware control |
| 2 | MCLK | Digital input | Master clock:256,384,512 or 768f s ( f s = word clock frequency) |
| 3 | BCLK | Digital input/output | Audio interface bit clock |
| 4 | LRCLK | Digital input/output | Audio left/right word clock |
| 5 | DVDD | Supply | Digital positive supply |
| 6 | DGND | Supply | Digital negative supply |
| 7 | DIN1 | Digital input | DAC channel 1 data input |
| 8 | DIN2 | Digital input | DAC channel 2 data input |
| 9 | DIN3 | Digital input | DAC channel 3 data input |
| 10 | DNC | Do not connect | Do not connect |
| 11 | ML/I2S | Digital input | Software Mode: Serial interface Latch signal Hardware Mode: Input Audio Data Format |
| 12 | MC/IWL | Digital input | Software Mode: Serial control interface clock Hardware Mode: Audio data input word length |
| 13 | MD/DM | Digital input | Software Mode: Serial interfacedata Hardware Mode: De-empahsis selection |
| 14 | MUTE | Digital input/output | DAC Zero flag output or DAC mute input |
| 15 | TESTREF1 | Digital input | Test pin |
| 16 | VREFN | Supply | Digital negative supply and substrate connection |
| 17 | VREFP | Supply | Digital positive reference supply |
| 18 | VMID | Analog output | Midrail divider decoupling pin; 10uf external decoupling |
| 19 | NC | Do not connect | Do not connect |
| 20 | NC | Do not connect | Do not connect |
| 21 | VOUT1L | Analog output | DAC channel 1left output |
| 22 | VOUT1R | Analog output | DAC channel 1right output |
| 23 | VOUT2L | Analog output | DAC channel 2 left output |
| 24 | VOUT2R | Analog output | DAC channel 2 right output |
| 25 | VOUT3L | Analog output | DAC channel 3 left output |
| 26 | VOUT3R | Analog output | DAC channel 3 right output |
| 27 | AGND | Supply | Analogue negative supply and substrate connection |
| 28 | AVDD | Supply | Analogue positive reference supply |

Note:

Digital input pins have Schmitt trigger input buffers.