FIRMWARE MANAGEMENT..

Продук; Аудио Системы; Revision No.; V710975. O/S; N/A. предме; Program Update for LH-TK250/550/750 Series. Main Chip; Version; EA0505063_0m. Firmware; Micom. Release Date; 2007-01-15. Option Code; 01 02 03 04 05 06 07 08 09 10 11 12 13 14 15 16.. Program Part No.; 6957R-753AA; System Model. содержание; #.It was improvement of performance; Improved playability for DVD-RW. #. Firmware Version : EA0505063_0m #. Countermeasure : Updating by using the newest firmware. Refer to the attatchment for updating method. # Program upgrade method [Current system information display] Push the button on remote controller as followed sequence Setup -> TV Aspect -> Select 16:9 -> Setup -> Setup -> 1 3 9 7 1 3 9 -> Enter [CD Write method] 1. Rename the file name to "BANK30.ROM" e.g.) HK44610200E1_EA0505063_0m.ROM -> BANK30.ROM 2. The update program (BANK30.ROM) the write it does in Root directory of Blank CD. (Recommended CD recording software : Roxio Easy CD Creator, Nero Burning Rom, etc...) [ Update method ] 1. First of all, Turn on the Set and then Insert the disk in the set. 2. When the CD is recognized, in the screen " Flash" The letter which is comes out and It reads, it erases, it writes it repeats.(this time, It does not press the power button roll up!) 3. When the write is over " Done" The message which is comes out. 4. It draws out the cord and to insert when it turns on the power switch and the update is completed.. Apply Model; Factory; Factory Model; Suffix; Модель; Factory; Factory Model; Suffix; Модель. EAIN; LH-TK250SC; AA3RMLS; LH-TK250X; ECHZ; LH-TK250SC; AA3RMLQ; LH-TK250Q. ECHZ; LH-TK250SC; AA3RMLQ; LH-TK250Q; ECHZ; LH-TK250SC; AA3RMLX; LH-TK250X. ECHZ; LH-TK250SC; AA3RMLX; LH-TK250X; ECHZ; LH-TK250SC; AA6KALX; LH-TK250X. ECHZ; LH-TK250SC; AA8UKLQ; LH-TK250Q; ECHZ; LH-TK250SC; AA8UKLX; LH-TK250X. ECHZ; LH-TK250SC; AA8ULLX; LH-TK250Q; ECHZ; LH-TK250SC; AA9LPLD; LH-TK250D. ECHZ; LH-TK250SC; AA9LPLD; LH-TK250D; ECHZ; LH-TK255SD; AA3RMLQ; LH-TK255Q. ECHZ; LH-TK255SD; AA3RMLX; LH-TK255X; ECHZ; LH-TK255SD; AA8UKLQ; LH-TK255Q. ECHZ; LH-TK255SD; AA8UKLX; LH-TK255X; ECHZ; LH-TK255SD; AA8ULLX; LH-TK255Q. EAIN; LH-TK551SB; AA3RMLX; LH-TK550X; EAIN; LH-TK551SB; AA3RMLX; LH-TK550X. EAIN; LH-TK551SB; AA6KALX; LH-TK550X; EAIN; LH-TK551SB; AA8UKLX; LH-TK550X. EAIN; LH-TK551SB; AA8UKLX; LH-TK550X; ECHZ; LH-TK551SB; AA3RMLQ; LH-TK550Q. ECHZ; LH-TK551SB; AA3RMLX; LH-TK550X; ECHZ; LH-TK551SB; AA3RMLX; LH-TK550X. ECHZ; LH-TK551SB; AA6KALX; LH-TK550X; ECHZ; LH-TK551SB; AA8UKLQ; LH-TK550Q. ECHZ; LH-TK551SB; AA8UKLX; LH-TK550X; ECHZ; LH-TK551SB; AA8UKLX; LH-TK550X. EAIN; LH-TK551TA; AA3RMLX; XH-TK555X; EAIN; LH-TK551TA; AA6KALX; XH-TK555X. EAIN; LH-TK551TA; AA8UKLX; XH-TK555X; ECHZ; LH-TK551TA; AA3RMLX; XH-TK555X. ECHZ; LH-TK551TA; AA6KALX; XH-TK555X; ECHZ; LH-TK551TA; AA8UKLQ; XH-TK555Q. ECHZ; LH-TK551TA; AA8UKLX; XH-TK555X; EAIN; LH-TK551TB; AA3RMLS; XH-TK550X. ECHZ; LH-TK551TB; AA3RMLQ; XH-TK550Q; ECHZ; LH-TK551TB; AA3RMLX; XH-TK550X. ECHZ; LH-TK551TB; AA6KALX; XH-TK550X; ECHZ; LH-TK551TB; AA8UKLQ; XH-TK550Q. ECHZ; LH-TK551TB; AA8UKLX; XH-TK550X; ECHZ; LH-TK551TB; AA8UKLX; XH-TK550X. EAIN; LH-TK751TA; AA3RMLX; XH-TK755X; EAIN; LH-TK751TA; AA6KALX; XH-TK755X. EAIN; LH-TK751TA; AA8UKLX; XH-TK755X; ECHZ; LH-TK751TA; AA3RMLQ; XH-TK755Q. ECHZ; LH-TK751TA; AA3RMLX; XH-TK755X; ECHZ; LH-TK751TA; AA6KALX; XH-TK755X. ECHZ; LH-TK751TA; AA8UKLQ; XH-TK755Q; EAIN; LH-TK751TB; AA3RMLS; XH-TK750X. EAIN; LH-TK751TB; AA3RMLX; XH-TK750X; EAIN; LH-TK751TB; AA3RMLX; XH-TK750X. ECHZ; LH-TK751TB; AA3RMLQ; XH-TK750Q; ECHZ; LH-TK751TB; AA3RMLX; XH-TK750X. ECHZ; LH-TK751TB; AA3RMLX; XH-TK750X; ECHZ; LH-TK751TB; AA6KALX; XH-TK750X. ECHZ; LH-TK751TB; AA8UKLX; XH-TK750X; ECHZ; LH-TK751TB; AA8UKLX; XH-TK750X. ECHZ; LH-TK751TF; AA3RMLQ; XH-TK756Q; ECHZ; LH-TK751TF; AA3RMLX; XH-TK756X. ECHZ; LH-TK751TF; AA3RMLX; XH-TK756X; ECHZ; LH-TK751TF; AA8UKLQ; XH-TK756Q. ECHZ; LH-TK751TF; AA8UKLX; XH-TK756X; ECHZ; LH-TK751TF; AA8UKLX; XH-TK756X... Applicable Buyer; Имя покупателя; Код покупателя; Имя покупателя; Код покупателя; Имя покупателя; Код покупателя. LGEAK; KZ000001; ALMATY; KZ000002; MOSCOW; RU000001. PETERSBURG; RU000002; VLADIVOSTOK; RU000103; LGERI; RU000549. LGEUA; UA000102; LGEUR; UA000523... Attach; HK44610200E1_EA0505063_0m.zip...