1.	1. ELECTRICAL REQUIREMENTS	2

A.	LCD Panel Specification	2

B.	Power Management	3

C.	Display Modes FOR Inspections	5

2.	ADJUSTMENT CONDITIONS	6

A.	Measuring Apparatuses Used	6

B.	Input Signal	6

C.	Indication	6

3.	ADJUSTMENT of Power Supply	6

4.	DISPLAY CONTROL BOARD	7

A.	Description	7

B.	Features	7

C.	Block Diagram (Control CKT)	7

D.	Connector Locations	8

E.	Connector Type	8

F.	Connector pin assignment	8

5.	VK-563 Control Panel Board	11

A.	Description	11

B.	Connector and Switch Locations	11

C.	Connector type	11

D.	Connector pin Assignment	12

6.	VP-570 POWER Board	14

A.	Description	14

B.	Features	14

C.	Interface Signals	14

D.	Electrical Specification:	15

E.	SAFETY	17

7.	CIRCUIT DESCRPTION	18

A.	Micro-Controller Circuit	18

B.	Circuit of Plug and Play	18

C.	System Clock	18

D.	Image Engine (Zoom)	18

E.	Power Regulator	18

8.	INTRODUCTION	19

A.	Front Panel Control and Led	19

B. Rear Panel connector Input Signals	20

9.	TROUBLESHOOTING	21

A.	Main Procedure	21

APPENDIX A: ES558 P.C.B.A ASSEMBLY	26

APPENDIX B: ES558 DISPLAY UNIT ASSEMBLY	38

# 1. ELECTRICAL REQUIREMENTS

## LCD Panel Specification

| Display: | 15 inch (15” viewable image size): active matrix: thin film transistor (TFT): liquid crystal display (LCD): 0.297 mm dot pitch: R.G.B. Vertical stripe 200 cd//m2 for HD2(250 cd//m2 for HD1) white luminance, typical: 300:1 contrast ratio, typical |
| --- | --- |
| Compatibility: | 640 x 350: VGA-350 720 x 400: VGA text 640 x 400: VGA-GRAPH 640 x 480: VGA, 60Hz to 75 Hz vertical refresh rate 800 x 600: 56Hz to 75 Hz vertical refresh rate 1024 x 768 non-interlaced: 60Hz to 75 Hz vertical refresh rate |
| Synchronization Frequencies: | Horizontal: 24 kHz to 61 kHz Vertical: 55 Hz to 75 Hz (1024 x 768 is up to 75 Hz) Pixel Frequency: 21 Mhz to 78 Mhz |
| Resolution: | Horizontal: 1024 dots Vertical: 768 lines |
| Active Display Area: | Horizontal: 304.1 mm Vertical: 228.1 mm |
| Viewing Angles: | Hannstar Up 45deg down 55deg (TYP) Left 65deg, Right 65deg (TYP) CR > 10 |
| Display Colors: | 6-bits driver |
| Power Supply: | AC 100 to 240V worldwide input, 50 / 60Hz |
| Power Consumption: | Typical: 25W on mode +10 / -25% |
| Environmental: | Operating temperature: 0ºC to 50ºC Storage temperature: -20ºC to 60ºC Storage humidity: 10%~90% |

## Power Management

### Power Management condition and status

| State |  | Signals |  | Power | Video | LCD | LED |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal | Vertical | Video | Supply | Circuit |  |  |
| On | Pulses | Pulses | Active | On | On | On | Green |
| Stand-by | No Pulses | Pulses | Blanking |  | Off | Off | Amber |
| Suspend | Pulses | No Pulses |  |  |  |  |  |
| Off | No pulses | No pulses | Blacking | On |  |  | Amber |
| DC Power off | Don’t care | Don’t care | Don’t care | Off | Off | Off ff o | Off |

### LED definition Table

| State | LED |
| --- | --- |
| On mode | Green |
| Stand-by | Amber |
| Suspend | Amber |
| Off mode | Amber |
| power off | Dark |
| disconnection | 1. Amber (stand-by; suspend; off mode) 2. LED Dark (power off) |

### Power Consumption

Meet VESA DPMS Proposal

| On mode | 30 Wmax |
| --- | --- |
| Stand-by | 3 Wmax |
| Suspend | 3 Wmax |
| Off mode | 3 Wmax |
| DC power off | 3 Wmax |
| disconnection | 3 Wmax |

- Measured from AC input, and not include audio at power-saving state

- The stand-by, suspend and off mode recover to on mode about 3 seconds.

## Display Modes FOR Inspections

### Supported Timing(for Not supported timing will go to power  saving mode)T

| TIMING | FH(KHZ) FV(HZ) | SYNC POLARITY | TOTAL (DOT/LINE) | ACTIVE (DOT/LINE) | SYNC WIDTH (DOT/LINE) | FRONT PORCH (DOT/LINE) | BACK PORCH (DOT/LINE) | PIXEL FOREQ. (MHZ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 640x350 VGA-350 | 31.469 |  | 800 | 640 | 96 | 16 | 48 | 25.175 |
|  | 70.087 |  | 449 | 350 | 2 | 37 | 60 |  |
| 640x400 NEC PC9801 | 24.83 |  | 848 | 640 | 64 | 64 | 80 | 21.05 |
|  | 56.42 |  | 440 | 400 | 8 | 7 | 25 |  |
| 640x400 VGA-GRAPH | 31.469 |  | 800 | 640 | 96 | 16 | 48 | 25.175 |
|  | 70.087 |  | 449 | 400 | 2 | 12 | 35 |  |
| 640x400 NEC PC9821 | 31.5 |  | 800 | 640 | 64 | 16 | 80 | 25.197 |
|  | 70.15 |  | 449 | 400 | 2 | 13 | 34 |  |
| 640x480 VGA-480 | 31.469 |  | 800 | 640 | 96 | 16 | 48 | 25.175 |
|  | 59.94 |  | 525 | 480 | 2 | 10 | 33 |  |
| 640x480 VESA-480-72Hz | 37.861 |  | 832 | 640 | 40 | 16 | 120 | 31.5 |
|  | 72.809 |  | 520 | 480 | 3 | 1 | 20 |  |
| 640x480 VESA-480-75Hz | 37.5 |  | 840 | 640 | 64 | 16 | 120 | 31.5 |
|  | 75 |  | 500 | 480 | 3 | 1 | 16 |  |
| 720x400 VGA-400-TEXT | 31.469 |  | 900 | 720 | 108 | 18 | 54 | 28.322 |
|  | 70.087 |  | 449 | 400 | 2 | 12 | 35 |  |
| 800x600 SVGA | 35.156 |  | 1024 | 800 | 72 | 24 | 128 | 36 |
|  | 56.25 |  | 625 | 600 | 2 | 1 | 22 |  |
| 800x600 VESA-600-60Hz | 37.879 |  | 1056 | 800 | 128 | 40 | 88 | 40 |
|  | 60.317 |  | 628 | 600 | 4 | 1 | 23 |  |
| 800x600 VESA-600-72Hz | 48.077 |  | 1040 | 800 | 120 | 56 | 64 | 50 |
|  | 72.188 |  | 666 | 600 | 6 | 37 | 23 |  |
| 800x600 VESA-600-75Hz | 46.875 |  | 1056 | 800 | 80 | 16 | 160 | 49.5 |
|  | 75 |  | 625 | 600 | 3 | 1 | 21 |  |
| 1024x768 XGA | 48.363 |  | 1344 | 1024 | 136 | 24 | 160 | 65 |
|  | 60.004 |  | 806 | 768 | 6 | 3 | 29 |  |
| 1024x768 COMPAQ-XGA | 53.964 |  | 1328 | 1024 | 176 | 16 | 112 | 71.664 |
|  | 66.132 |  | 816 | 768 | 4 | 8 | 36 |  |
| 1024x768 VESA-768-70Hz | 56.476 |  | 1328 | 1024 | 136 | 24 | 144 | 75 |
|  | 70.069 |  | 806 | 768 | 6 | 3 | 29 |  |
| 1024x768 VESA-768-75Hz | 60.023 |  | 1312 | 1024 | 96 | 16 | 176 | 78.75 |
|  | 75.029 |  | 800 | 768 | 3 | 1 | 28 |  |

# ADJUSTMENT CONDITIONS

## Measuring Apparatuses Used

Necessary measuring equipment that for the adjustment of LA-1221JMW’s MAIN PWB are following items.

Adjusted Video Generator.

Oscilloscope (more than 4 channels required)

DC voltmeter

## Input Signal

Input signal by using video signal generator with VGA Card port for this adjustment.

## Indication

Input signal by using video signal generator with VGA Card port for this adjustment.

# ADJUSTMENT of Power Supply

- Input voltage					AC 100 ~ 240Vac

- Rated input voltage				AC 90 ~ 264Vac

- Frequency						50 / 60 Hz

- Rated frequency				47 ~ 63 Hz

- DC output

| Out voltage | +12V | +5V |
| --- | --- | --- |
| Maximum output current | +1.5A | +1.5A |
| Minimum output current | 0A | +0.1A |
| Range of voltage Regulation | +25%~-5% | +5%~-3% |

# DISPLAY CONTROL BOARD

## Description

The VL-565 display control board is design to directly convert the R.G.B signals from VGA Card D-sub port to optimum LCD timing signals so as to construct a high display quality LCD monitor.

## Features

- On board Myson micro-p chip to detect display timings and control user functions.

- Using M-STAR chip to convert R.G.B signals to digital timing

- Using M-STAR chip to offer full screen expansion function on non-XGA mode(Auto-Zoom).

- Support VESA DPMS function.

- Support DDC2B functions.

## Block Diagram (Control CKT)

System Level Interface with Integrated Timing Controller

( VL-565 Diagram – Hannstar Panel solution)

### M-STAR  Operation

The MST9015 is highly-integrated display processors. It has an ADC/PLL block on a single chip. The cost-effective MST9015 is suitable for the XGA. Other features include a brand new high-end scaler, Active Color Management, a flexible OSD, Autoadjust, SureSyncTM and independent monitoring of the analog interface. A new standalone on-chip pattern generator has been incorporated to simplify the manufacture of monitors.

## Connector Locations

( VL-565 Control Board for Hannstar Panel solution )

## Connector Type

Location	Type	Maker	Number of pins

CN6	E&T 4500-13	E&T	13

CN1	E&T 4500-08	E&T	9  (W/O Audio is 6 pin)

CN4	E&T 4500-09	E&T	8

CN3	AF740B-A2G1T	P-TWO	40

CN2	AF740B-A2G1T	P-TWO	40

CN8	E&T 4500-10	E&T		10 (W/I Audio only)

## Connector pin assignment

### CN1

- Pin NO.	Signal

- 1	Vertical Sync

- 2	Horizontal Sync

- 3	Blue Video Ground

- 4	Blue Video

- 5	Green Video Ground

- 6	Green Video

- 7	Red Video Ground

- 8	Red Video

- 9	Shield Ground

- 10	DDC-SCL

- 11	DDC-SDA

- 12	PC-DETECT

- 13	PC-5V

### ( VL-565 Control Board for Hannstar Panel )

### F-3   CN4

- Pin NO.	Signal	Comment

- 1	KEY-FUN(+)	Function select counter-clockwise key

- 2	LED-RO	Power saving mode

- 3	LED-GO	Monitor is ON

- 4	GND	GND

- 5	KEY-Power	Power ON/OFF signal

- 6	KEY-Fun(-)	Function select anti-clockwise key

- 7	KEY-VOL(+)	Adjust up key

- 8	KEY-VOL(-)	Adjust down key

### F-4   CN1(W/O Audio IS 6pin)

- Pin NO.	Signal	Comment

- 1	+5V	VL547 / VL-542 power input

- 2	+5V	VL547 / VL-542 power input

- 3	GND	Ground

- 4	GND	Ground

- 5	BLON	Back Light ON/OFF.

- 6	BLADJ	Brightness Adjustment.

- 7	NONE

- 8	+5V	For Audio Power

- 9	AGND	Audio GND

# VK-563 Control Panel Board

## Description

- The VK-563 is designed to offer an user interfaced control panel which passes and receives signals to and from power board .

## Connector and Switch Locations

## Connector type

- Location	Type	Maker	Number of pins

- J1	on-board harness	Great Land	10	(Audio Function Only)

- J2	SCJ-0348-C	SC	9	(Audio Function Only)

- J3	87502-0200	ACES	2	(Audio Function Only)

- J4	87502-0200	ACES	2	(Audio Function Only)

- J5	on-board harness	Great Land	8

- VR1	F1001G-3 B50K	FAVOR U	5	(Audio Function Only)

## Connector pin Assignment

### J1  (Audio Function Only)

- Pin NO.	Signal	Comment

- 1	L+	Audio Out L+

- 2	L-	Audio Out L-

- 3	R+	Audio Out R+

- 4	R-	Audio Out R-

- 5	GAIN	Audio GAIN adjust

- 6	VRH	VR PULL Hight

- 7	GND	GND

- 8	HP-L	EAR Phone OUT L

- 9	HP-LR	EAR Phone OUT R

- 10	HP-S	EAR Phone signal control

### J2  (Audio Function Only)

- Pin NO.	Signal

- 1	GND

- 2	EAR Phone OUT L

- 3,7	EAR Phone OUT R

- 4	NC

- 5	NC

- 6	NC

- 8	EAR Phone signal control

- 9	NC

### J3 & J4  (Audio Function Only)

Pin NO.	Signal	Comment

1	OUTL(OUTR)	Speaker out

2	GND	GND

### J5

Pin NO.	Signal	Comment

- NONE	Function Select counter-clockwise key

- LED-RO	Power saving mode.

3	LED-GO	Monitor on mode.

4,10	GND	Ground

5	KEY-POWER	Power OF/OFF signal

6	KEY-FUN(-)	Anti-clock wise key

7	KEY-VOL(+)	Adjust Up Key

8	KEY- VOL(-)	Adjust Down Key

### VR1  (Audio Function Only)

- Volume control , clockwise for increasing ; counter-clockwise for decreasing

### Switch definition

- Location	Definition

- S1	Function select by clockwise direction

- S2	Adjust down

- S3	Adjust up

- S4	Function select by counter-clockwise direction

- S5	Power ON/OFF

### LED definition

- Location	Definition

- D1	Green for ON mode; Amber for DC power off;  stand by;

- suspend and off mode.

# VP-570 POWER Board

## Description

This specification defines the performance and characteristic for the VP-570 power board. It supplies the following items :

1).A constant 5V output source for logic board.

2).A constant 5V output source for audio’s power source.

2).A constant 12V output source for Inverter’s power source.

## Features

Wide input range : AC input 90Vrms ~ 264Vrms

High efficiency  : η>75%

Inverter brightness adjustment by continual mode

Protection function : DC output is design by auto-recovery.

It include OPP ,SCP.

## Interface Signals

### Input

1.AC Inlet: RICHBAY R-301-90

2.J701 :SCJ-0345-I-X-S	SC	3Pin

### output

J801 : The wire set of  Pin2.0 /7Pin to Pin1.25/7 Pin , 110mm length

| PIN NO. | Function | Function |
| --- | --- | --- |
| 1 | +5V | +5V supply for logic board |
| 2 | +5V | +5V supply for logic board |
| 3 | GND | Ground |
| 4 | GND | Ground |
| 5 | VEN | Enable signal for inverter (Low active , <1V) |
| 6 | VBRI | Brightness signal for inverter (0V to 3.3V) |
| 7 | -- |  |
| 8 | +5V | +5V supply for audio’s power |
| 9 | GND | Audio ground |

J901,J902 : Interver High voltage output

| PIN NO. | Function | Comment |
| --- | --- | --- |
| 1 | LV | Low voltage (common) |
| 2 | HV | High voltage for lamp |

## Electrical Specification:

### AC-DC Electrical specification

#### Input Specification

| No | Item | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Input Voltage | ----- MAX | 100 | --- | 240 | Vac |
| 2 | Input Frequency | ----- 1.5A | 47 | --- | 63 | Hz |
| 3 | Input Current | 100Vac@ Type load 1.5A | --- | --- | 1.0 | Arms |
| 4 | Inrush Current | Cold Start @Vin=100Vrms | --- | --- | 30 | A0-P |
|  |  | Cold Start @Vin=220Vrms |  | --- | 50 | A0-P |
| 5 | Hold Up Time | @full load & 100Vac input and Type load | 10 | --- | --- | ms |
| 6 | Turn on time | Vin =110Vac | --- | 1.0 | --- | S |
| 7 | Efficiency | +12V/1.5A ,+5Vcc/1.5A | 70 | --- | --- | % |
| 8 | Input Power Consumption | Vin=220Vac | --- | --- | 1.2 | W |

*Cold start up of Inrush Current : Power board off(keep over 1 hour) and Ta=25   degree C

#### AC-DC Output Specification

- Output

|  | Tolerance | Output Current |  | Voltage Tolerance |
| --- | --- | --- | --- | --- |
| Output Voltage | - | MIN | MAX |  |
| +5Vdc | +5 /-3 % | 0.05A | 1.5A | 4.85~5.25V dc |
| +12Vdc | +25/-5 % | 0A | 1.5A | 11.4~15Vdc |
| Ripple | - Measured at DC output terminals which are paralleled with a 10uf Ecap &0.1uf Cermic cap. 2.Band width is limited within 20MHz. | --- | 1% | +5Vdc:50mVp-p +12Vdc:120mVp-p |
| Noise |  | --- | 3% | +5Vdc:150mVp-p +12Vdc:360mVp-p |
| Dynamic Load Regulation | 50~100% or 100~50% load change of any DC output @50% duty of 1MHz (min.) | --- | 5% | --- |
| Over / Under Shoot | @ Power line on/off | --- | 5% |  |

- *+5Vcc load regulation test: the +12V loading at 1.0A

*+12Vcc load regulation test: the +5V loading at 1.0A

#### Protection Function: Auto-recovery design is acceptable, but input power must be less than 5W

1)SCP: Short circuit protection must be acted on both outputs

2)OPP: Should be protected when output power consumption is within 40W ~ 70W

3)OCP:

|  | Min. | Max. | Test condition |
| --- | --- | --- | --- |
| +5Vdc(for logic) | 2A | 6A | +12V @1.5A |
| +12Vdc | 1.8A | 4.5A | +5V(for logic) @1.5A |

### Inverter Electrical Specification:

For Hannstar HSD150SX84-B

|  | Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Input Voltage | --- | --- | 12 | --- | V |
| Input Current | --- | --- | 1.0 | --- | A |
| Backlight ON/OFF Control | ON | --- | 0 | --- | V |
|  | OFF o | --- | 3.0 | --- | V |
| Brightness Adjust (Lamp Current Control) | Min. Luminance / Max. Luminance | --- | 55% | --- | --- |
| Output Voltage | Vin=12V, Iout=6.5mA | --- | 750 | --- | Vrms |
| Brightness | HSD150SX84-B | 200 | 250 | --- | Cd/m² |
|  | HSD150SX84-D | 175 | 200 | --- |  |
| Output Current(Each connector) | Vbri=0V~3.3V | 3 | 6.5 | 7 | mA |
| Frequency | --- | 40 | 50 | 70 | KHz |
| Open Lamp Voltage | --- | 1350 | --- | --- | Vrms |
| Striking Time | --- | --- | 1.0 | --- | S |
| Lamp Current Balance | --- | --- | ±0.3 | --- | mA |
| Efficiency | Vin=12V | --- | 80 | --- | % |
| Operating Life Time | --- | 30000 | --- | --- | Hr |

*.The open lamp voltage is testes at output connector terminal

*.Life time(Hr) can be defined as the time in which it continues to operate under the condition: 
Ta=253C, IL=6.0Ma(ms) and fL=30 KHz until one of the following event occurs:

1. When the brightness becomes 50%

2. When the startup voltage (Vs) at 0C becomes higher than the maximal value of Vs specified above

## SAFETY

### Leakage Current: 0.25mA @ 100Vac

### Insulation Resistance: more than 3M ohms while with attending a voltage of 500Vac

### Hi-Pot : 3Kvac with using 3mA cut off current

# CIRCUIT DESCRPTION

## Micro-Controller Circuit

The U4 (for VL-565 ) (MTV312M64) is a MICRO-Pcontroller.

The Y1 is 14.318MHz XTAL.

## Circuit of Plug and Play

Plug and play allows the serial communication of host PC and peripherals offering minimal configurations to end users.  This monitor supports DDC2B communication protocal.

SDATA and VCLK are input to U4 (for VL-565 ) (Build in micro-p).

## System Clock

The Y1 (14.318MHz XTAL) supports U4&U1 (for VL-565 ) clock.

## Image Engine (Zoom)

The U1 (for VL-565 ) (M-STAR) is a image engine that have following functions.

(1) The M-STAR are both high guilty scalar and timing controller chip.

## Power Regulator

- The U7 (for VL-565 ) (APL1084) is 5A linear regulator that transfer voltage from 5V to 3.3V supports U1 and Panel Vcc.

- The U6 (for VL-565 ) (APL1117) is 1A linear regulator that transfer voltage from 5V to 2.5V.

# INTRODUCTION

## Front Panel Control and Led

ES558(Without Audio)

| Front Panel Controls |  |  |
| --- | --- | --- |
| Item | Control | Function |
| 1 | DC Power Switch | Press the power switch to switch the monitor ON/OFF. |
| 2 | DC Power-On Indicator | LED lights Green color --- Power is ON. LED lights Yellow --- Monitor is in "Power Saving Mode". LED is off --- Power is OFF. |
| 3 | Function Select Buttons | Press either left or right control button for OSD (On Screen Display) menu selection. |
| 4 | Adjustment Control Buttons | Press the left button to decrease the OSD setting and press the right button to increase the OSD setting. |

## B. Rear Panel connector Input Signals

| Rear Panel Cable and Connector |  |  |
| --- | --- | --- |
| Item | Cable / Connector | Function |
| 1 | Signal Cable | Connect the video port |
| 2 AC IN | Power Connect | Connect the AC power cord |
| 3 | Audio connector (ES555 Only) | Connect the audio port |

# TROUBLESHOOTING

## Main Procedure

### Power Circuit Troubleshooting

### Backlights Troubleshooting

### Performance Troubleshooting

### Function Troubleshooting