1. Set the PC to static IP. (ex.192.168.1.100)

2. Use console make the device into boot mode.

3. Execute “tftpd32.exe” (Put the firmware file in the tftpd32 folder first.)

4. Type “ tftp 192.168.1.1 192.168.1.100 router” in console

5. When the firmware upload is complete, type “reboot”

.

6. After reboot, type username/password：guest/guest, and type “config reset”.

7. After reset default, firmware upload is completed.