5. Power Supply section

The BATT lead is connected to the car battery, D363 is added to prevent damage when the power

supply polarity should accidently be reversed. It provides backup to IC701 for clock time (pin 12,

BACKUP) and via Q706 (6V regulator) and D713 for preset memory. When this lead is not connected,

e.g. when the set is taken out of its bracket, the built-in Lithium battery (BT1) provides backup via

D711. However, only preset memory is retained, the clock oscillator is stopped to reduce power

consumption.

The ACC lead is connected to the power switch, the power amplifier, Q361 and D357. Because of

D357 the set will operate properly if only the ACC lead is connected to a power supply. When the

power is switched on, the reset circuit (IC701, Q751 and Q363) will activate the muting for a brief

moment. IC701 can also activate the muting (e.g. during auto tuning) via Q364.

Q355, Q356 and Q357 are parallelled to form a +9V regulator which is switched on by Q358 and

Q360. It provides power to different circuits in the set, controlled by switching signals from IC701

(pin 25, AM; pin 24, FM). The deck board receives its power directly from the ACC lead via Q361.

Q361 is switched on by Q362 which in turn is switched by the TAPE +B signal from IC701 (pin 50).

When a certain circuit is not working, it is a good habit to check the relevant power supply transistor

and its switch-on signal because power supply problems mostly affect only part of the set.