## Справочник MOSFET. AO3415

- БИПОЛЯРНЫЕ

- MOSFET

- IGBT

- SCR

- SMD-КОДЫ

- КОРПУСА

- СКАЧАТЬ

Начало формы

Конец формы

## AO3415 MOSFET - описание производителя. Даташиты. Основные параметры и характеристики. Поиск аналога. Справочник

Наименование прибора: AO3415

Тип транзистора: MOSFET

Полярность: P

Максимальная рассеиваемая мощность (Pd): 1.5 W

Предельно допустимое напряжение сток-исток (Uds): 20 V

Предельно допустимое напряжение затвор-исток (Ugs): 8 V

Пороговое напряжение включения Ugs(th): 0.9 V

Максимально допустимый постоянный ток стока (Id): 4 A

Максимальная температура канала (Tj): 150 °C

Время нарастания (tr): 9 ns

Выходная емкость (Cd): 115 pf

Сопротивление сток-исток открытого транзистора (Rds): 0.041 Ohm

Тип корпуса: SOT23-3

Аналог (замена) для AO3415

BUY TRANSISTORS

## AO3415 Datasheet (PDF)

1.1. ao3415a.pdf  Size:319K _aosemi

﻿AO3415A 20V P-Channel MOSFET General Description Product Summary VDS -20V The AO3415A uses advanced trench technology to provide excellent RDS(ON), low gate charge and operation ID (at VGS=-4.5V) -5A with gate voltages as low as 1.8V. This device is suitable RDS(ON) (at VGS= -4.5V) < 41mΩ for use as a load switch applications. RDS(ON) (at VGS= -2.5V) < 53mΩ RDS(ON) (at VGS= -

1.2. ao3415.pdf  Size:283K _aosemi

﻿AO3415 20V P-Channel MOSFET General Description Product Summary VDS -20V The AO3415 uses advanced trench technology to provide excellent RDS(ON), low gate charge and operation with gate ID (at VGS=-4.5V) -4A voltages as low as 1.8V. This device is suitable for use as RDS(ON) (at VGS= -4.5V) < 41mΩ a load switch applications. RDS(ON) (at VGS= -2.5V) < 53mΩ RDS(ON) (at VGS= -1.

1.3. ao3415w.pdf  Size:1656K _kexin

﻿SMD Type MOSFET P-Channel MOSFET (KO3415W) AO3415W ■ Features ● VDS (V) =-20V ● ID =-4A (VGS =-4.5V) ● RDS(ON) ＜ 50mΩ (VGS =-4.5V) ● RDS(ON) ＜ 60mΩ (VGS =-2.5V) ● RDS(ON) ＜ 75mΩ (VGS =-1.8V) 1 Gate 2 Source 3 Drain ■ Absolute Maximum Ratings Ta = 25℃ Parameter Symbol Rating Unit Drain-Source Voltage VDS -20 V Gate-Source Voltage VGS ±8 Conti

1.4. ao3415a.pdf  Size:1729K _kexin

﻿SMD Type MOSFET P-Channel MOSFET AO3415A (KO3415A) SOT-23 Unit: mm +0.1 2.9-0.1 +0.1 0.4 -0.1 ■ Features 3 ● VDS (V) =-20V ● ID =-5 A (VGS =-4.5V) ● RDS(ON) ＜ 43mΩ (VGS =-4.5V) 1 2 +0.1 0.95-0.1 0.1+0.05 ● RDS(ON) ＜ 55mΩ (VGS =-2.5V) -0.01 +0.1 1.9-0.1 ● RDS(ON) ＜ 75mΩ (VGS =-1.8V) ● RDS(ON) ＜ 100mΩ (VGS =-1.5V) 1. Gate 2. Source D 3. Dra

1.5. ao3415as.pdf  Size:1662K _kexin

﻿SMD Type MOSFET P-Channel MOSFET AO3415AS (KO3415AS) SOT-23-3 Unit: mm +0.2 2.9 -0.1 +0.1 0.4-0.1 ■ Features 3 ● VDS (V) =-20V ● ID =-4A (VGS =-4.5V) ● RDS(ON) ＜ 45mΩ (VGS =-4.5V) 1 2 +0.02 +0.1 0.15 -0.02 0.95 -0.1 D ● RDS(ON) ＜ 54mΩ (VGS =-2.5V) +0.1 1.9 -0.2 ● RDS(ON) ＜ 75mΩ (VGS =-1.8V) ● ESD Rating: 3000V HBM G 1. Gate 2. Source 3. Dra

1.6. ao3415a-3.pdf  Size:1694K _kexin

﻿SMD Type MOSFET P-Channel MOSFET AO3415A (KO3415A) SOT-23-3 Unit: mm +0.2 2.9 -0.1 +0.1 0.4 -0.1 3 ■ Features ● VDS (V) =-20V ● ID =-5 A (VGS =-4.5V) 1 2 ● RDS(ON) ＜ 43mΩ (VGS =-4.5V) +0.02 +0.1 0.15 -0.02 0.95 -0.1 +0.1 1.9 -0.2 ● RDS(ON) ＜ 55mΩ (VGS =-2.5V) ● RDS(ON) ＜ 75mΩ (VGS =-1.8V) ● RDS(ON) ＜ 100mΩ (VGS =-1.5V) 1. Gate 2. Source

1.7. ao3415.pdf  Size:1982K _kexin

﻿SMD Type MOSFET P-Channel MOSFET AO3415 (KO3415) SOT-23 Unit: mm +0.1 2.9-0.1 +0.1 0.4 -0.1 ■ Features 3 ● VDS (V) =-20V ● ID =-5 A (VGS =-4.5V) ● RDS(ON) ＜ 43mΩ (VGS =-4.5V) 1 2 +0.1 0.95-0.1 0.1+0.05 ● RDS(ON) ＜ 55mΩ (VGS =-2.5V) -0.01 +0.1 1.9-0.1 ● RDS(ON) ＜ 75mΩ (VGS =-1.8V) ● RDS(ON) ＜ 100mΩ (VGS =-1.5V) 1. Gate 2. Source D 3. Drain

1.8. ao3415-3.pdf  Size:1998K _kexin

﻿SMD Type MOSFET P-Channel MOSFET AO3415 (KO3415) SOT-23-3 Unit: mm +0.2 2.9-0.1 +0.1 0.4 -0.1 ■ Features 3 ● VDS (V) =-20V ● ID =-5 A (VGS =-4.5V) ● RDS(ON) ＜ 43mΩ (VGS =-4.5V) 1 2 ● RDS(ON) ＜ 55mΩ (VGS =-2.5V) +0.02 +0.1 0.15 -0.02 0.95 -0.1 +0.1 1.9-0.2 ● RDS(ON) ＜ 75mΩ (VGS =-1.8V) ● RDS(ON) ＜ 100mΩ (VGS =-1.5V) 1. Gate D 2. Source 3

1.9. ao3415.pdf  Size:1085K _shenzhen-tuofeng-semi

﻿Shenzhen Tuofeng Semiconductor Technology Co., Ltd AO3415 Rev 3: May 2004 AO3415 P-Channel Enhancement Mode Field Effect Transistor General Description Features The AO3415 uses advanced trench technology to VDS (V) = -20V provide excellent RDS(ON), low gate charge and ID = -4 A operation with gate voltages as low as 1.8V. This RDS(ON) < 45mΩ (VGS = -4.5V) device is suitable for

Adblock

Другие MOSFET... AO3403 , AO3404 , AO3404A , AO3406 , AO3407A , AO3409 , AO3413 , AO3414 , IRFP150N , AO3415A , AO3416 , AO3418 , AO3419 , AO3420 , AO3421 , AO3421E , AO3422====================================

. https://alltransistors.com/ru/mosfet/transistor.php?transistor=20377