COLOR TV RECEIVER

# C1425,C2121,C21R90,C2190,C21R90

## TEST & ALIGNMENT PROCEDURES

The Test & Alignment Procedures are applicable for EX chassis series for service and adjustment personnel’s reference.

Unless otherwise noted, the TV works in the PAL-B/G mode with PAL signal reception, OSD language is set to ENGLISH and programs options are 236.

TABLE OF CONTENTS

SEPARATE ADJUSTMENT

COARSE ADJUSTMENT OF WHOLE SET

FINE TUNING OF WHOLE SET

FINAL INSPECTION

APPENDIXⅠ TUNING MENU AND DATA OPTIONS

APPENDIXⅡ EEPROM PRE-WRITE-IN

APPENDIXⅢ MANUFACTURE MODE

Note: In the Procedures, the equipment for adjustment or inspection is required one unless otherwise noted.

ⅠSEPARATE ADJUSTMENT

1. Power Inspection

  - Equipment to Be Required

a. Digital voltmeter

b. Jig equipped with CRT

  - Inspection Condition

- Set the picture mode to Nature.

- Power source: 220V±5V AC

  - Adjustment Method

- Connect the chassis board to the jig.

- Power on the TV to inspect voltage.

  - For EX-1A chassis, inspect the following voltages.

Table 1

| Test Point | DC Voltage (V) |
| --- | --- |
| Negative of VD891 | 115.5V±1.5V |
| Negative of VD871 | 15V±1V |
| Negative of VD881 | 15V±1V |
| Positive of C877 | 8V±0.5V |
| Positive of C116 | 5V±0.5V |
| Positive of C876 | 5V±0.5V |
| Positive of C115 | 3.3V±0.3V |
| Positive of C453 | 10.5±1V |
| Positive of C462 | 16V±1V |
| Positive of C452 | 45V±3V |

  - For EX-1A1/1A2/1A4 chassis, inspect the following voltages.

Table 2

| Test Point | DC Voltage (V) |
| --- | --- |
| Negative of VD891 | 115.5V±1.5V |
| Negative of VD871 | 15V±1V |
| Negative of VD881 | 15V±1V |
| Positive of C477 | 8V±0.5V |
| Positive of C116 | 5V±0.5V |
| Positive of C476 | 5V±0.5V |
| Positive of C115 | 3.3V±0.3V |
| Positive of C453 | 10.5±1V |
| Positive of C462 | 16V±1V |
| Positive of C452 | 45V±3V |

Note:

It’s impossible to check the power part separately from the main chassis board as the part is mounted on the main chassis board. The power components, etc. should be checked for burnout when power-on. If burned out, do not power on the TV again until the cause is found out.

2. Tuning in

2.1 Equipment and Signal to Be Required

- Jig equipped with CRT

- Factory RC

- Oscilloscope

- Signal center

2.2 Adjustment Method

- Attenuate the signal to 60dBµV and input it to the ANTENNA IN terminal on the TV.

- Press the AUTO button on the factory RC and auto program begins. The following signals should be searched and memorized (See Table 3).

- After auto program, check if the picture and sound are normal. Under the condition of normal audio system, receive 1KHz mono audio signal and set the volume to 63. Measure voltage on two ends of the speaker to be not less than 10Vpp with the oscilloscope.

- Check size and color of characters for normality during tuning-in.

Note:

It's necessary to use the preset EEPROM before adjustment.

Table 3

| Channel No. | Receiving Channel | Test Pattern | Channel No. | Receiving Channel | Test Pattern |
| --- | --- | --- | --- | --- | --- |
| 1 | CH-01 | Color bar PAL D/K | 11 | CH-10 | Color card NTSC M |
| 2 | CH-03 | Color bar PAL D/K | 12 | CH-12 | Crosshatch pattern PAL D/K |
| 3 | CH-05 | Color card PAL D/K | 13 | Z-16 | White full-field pattern PAL D/K |
| 4 | Z-01 | Color bar NTSC M | 14 | Z-27 | Color bar PAL D/K |
| 5 | Z-02 | Toshiba pattern PAL D/K | 15 | Z-28 | Red full-field pattern PAL D/K |
| 6 | Z-03 | Blue full-field pattern PAL D/K | 16 | Z-37 | Color bar PAL B/G |
| 7 | Z-04 | Green full-field pattern PAL D/K | 17 | CH-13 | Red full-field pattern PAL I |
| 8 | Z-07 | Color bar PAL I | 18 | CH-21 | Toshiba pattern PAL D/K |
| 9 | CH-06 | Color card PAL D/K | 19 | CH-57 | Color bar PAL D/K |
| 10 | CH-08 | Color card SECAM B/G |  |  |  |

Ⅱ COARSE ADJUSTMENT OF WHOLE SET

1. Coarse Adjustment for Screen Voltage and Focus Voltage

1.1 Equipment to Be Required

- Factory RC

- Non-inductive screwdriver

1.2 Adjustment for Screen Voltage and Focus Voltage

- Receive CH-5 circle pattern and enter the TV into the USER mode (Note: VG2B differs depending on CRTs used in the M mode, which should be written in EEPROM.) (in the USER mode, brightness is set to 56 and contrast to 63.)

- Press the VSD button to turn off vertical output (Dark belts appear on the top of the screen and yellow “VG2: OUT LOW”, “VG2: OUT HIGH” or green “VG2: IN HIGH” is displayed on the bottom of the screen.)

- Adjust the screen VR on the FBT (lower on the FBT) so that green “VG2” on the bottom of the screen just skips to green“IN HIGH” from yellow“OUT LOW”.

- Adjust focus VR on the FBT (upper on the FBT) until good raster focus and high-definition picture are obtained.

2. Aging

2.1 Aging Setting

2.1.1 Equipment to Be Required

Factory RC

2.1.2 Setting Method

Set the volume to minimum, brightness, contrast and color to maximum and the TV to the M-MODE.

2.2 Aging Voltage and Aging Time

- Increase the rated operating voltage by 10%, to 242V (aging voltage).

- Ensure aging time not less than 30 minutes.

Ⅲ FINE TUNING OF WHOLE SET

1. Confirmation of All Voltages

  - Equipment and Signal to Be Required

- Digital DC voltmeter

- High frequency effective voltmeter

- Anode high-voltage testing equipment

1.2 Confirmation Method

- Check the resistors and coils for being burned. (If the fuse is burned out, do not power on the TV again until the cause is found out.)

- Receive CH-5 circle pattern and set the picture mode to Nature.

- Measure voltage of each test point with the DC voltmeter. Measure the filament voltage with the high-frequency effective voltmeter and CRT high voltage with the high-voltage testing equipment. The rated values are shown as below.

Table 4

| Test Point | Voltage Value (V) |
| --- | --- |
| Positive of C492 | 185V±5V |
| Filament pin of socket | 6.3V±0.3VRMS |
| 21” CRT anode | 25.5KV±1.2KV |
| Anode of 21” pure flat CRT (including Samsung, BMCC and Toshiba) | 27.5KV±1.5KV |
| Anode of other 21” pure flat CRTs | 26.5KV±1.5KV |
| Anode of 14” CRT (including IRICO, BMCC and Thai) | 22KV±1.3KV |
| Anode of 14” Lithuania CRT | 25KV±1.3KV |

Notes:

  - Measure voltage of the related isoelectronic point if it’s difficult to measure a certain point in Table 4.

  - Anode high voltage differs depending on CRTs used, which should be decided by engineers.

  - Spot check the voltage items included in Table 1 and Table 2, but excluded in Table 4.

2. Degaussing Coil Inspection

2.1 Inspection Equipment

Degaussing coil inspector

2.2 Inspection Method

- Remove the automatic degaussing coil. After it cools, remount the coil into the socket. Check degaussing.

- You may also use a magnetic sensor specific for degaussing inspection.

3. Adjustment for Screen Voltage and Focus Voltage

3.1 Equipment to Be Required

Factory RC

3.2 Adjustment Method

The same as Section 1.2 in “COARSE ADJUSTMENT OF WHOLE SET”.

4. Auto White Balance Adjustment

4.1 Adjustment Equipment

- Auto white balance adjuster

- Factory RC

4.2 Adjustment Method

- Set the brightness to MID, contrast to MAX and color to MIN with the Factory RC.

- Insert the wired connector connected with the auto white balance adjuster into the socket XS100 (or XS202 for EX-1A1) on the main chassis board.

- Control the adjuster and press the BUS OPEN button on the Factory RC to give an instruction to set the TV in the white balance adjustment mode.

- The adjuster automatically adjusts white balance. After completion, it gives an instruction to end the adjustment and reminds you of completion of adjustment on the screen. At this time, pull off the wired connector XS100 (or XS202 for EX-1A1).

- Press the BUS OPEN button again to turn off “BUS OPEN” on top of the screen.

Notes:

  - For EX-1A/ EX-1A1 chassis series, both dark balance and bright balance adjustments are required.

  - During white balance adjustment, set “COLOR TEMP” in the USER menu to Standard.

5. To Inspect Functions of Buttons on the TV or Remote Control

5.1 Inspection Equipment

- Remote control

- Signal input cable

  - Items Inspected

5.2.1 Inspected Buttons on the TV

- CHANNEL DOWN/UP (P-/+)

- VOLUME DOWN/UP (VOL-/+)

- MENU SELECT (MENU)

- TV/AV SELECT (TV/AV)

5.2.2 Inspected Buttons on the Remote Control

- STANDBY

- MUTE

- DIGIT SELECT

- AV/TV SELECT

- MENU SELECT (MENU, CHANNEL DOWN/UP and VOLUME DOWN/UP)

- Fast Operation (Red/Green/Yellow/Cyan buttons in the TV mode)

- ON SCREEN DISPLAY

- RECALL

- BIORHYTHM

- PICTURE MODE SELECT

- HELP

- SOUND MODE SELECT

- TELETEXT ON/OFF

- INDEX

- HOLD

- REVEAL

- ZOOM

- COLOR

- CANCEL

- MIX

- SUBCODE

- EAST/WEST

- AUTO

  - Inspection Method

    - To Inspect Functions of Buttons on the TV

- CHANNEL DOWN/UP buttons

Press the CHANNEL DOWN/UP buttons to change the channel number in the order of 1→2→3→ …… 235→0→1.

Note: The TV can search or memorize channel numbers up to 235. During auto program, the TV will skip cleared position numbers.

- VOLUME DOWN/UP buttons

Press the VOLUME DOWN/UP buttons to decrease or increase volume.

- MENU button

All the menus are displayed with dark blue frame and halftone back. And the selected characters are zoomed and displayed as cyan. Press the MENU button to call up the following menu in turn: PICTURESOUND (if with audio processing IC option)TIMESETUPTUNING (if in the TV mode)SWAPBIORHYTHMNo Menu (or SWAPHOTELBIORHYTHM if with the HOTEL function).

Press the MENU button to call up the primary menu. Press the CHANNEL DOWN/UP buttons to highlight a sub menu and VOLUME DOWN/UP buttons to access it.

PICTURE Menu

You may adjust the contrast/brightness/color/sharpness/tint via the PICTURE menu. “Tint” is available only when the signal system is set to NTSC. Press the CHANNEL DOWN/UP buttons to highlight an adjustment and VOLUME DOWN/UP buttons to adjust it. The adjustable tint ranges from –31 to +31, and other adjustments from 0 to 63. Press the MENU button to access the next menu. The PICTURE menu display disappears and the TV performs the corresponding operation if there is no operation or any other buttons than CHANNEL DOWN/UP and VOLUME DOWN/UP buttons are pressed within 6 seconds. During the PICTURE menu display, the TV detects the current color system if necessary. If the current color system is compelled to NTSC or the color system is set to Auto but with the actual color system NTSC, “Tint” is available in the PICTURE menu, otherwise not.

SOUND Menu

For TVs with audio processing IC, the SOUND menu is available. Call the SOUND menu. Press the CHANNEL DOWN/UP buttons to highlight one of adjustments (Volume, Bass, Treble and Balance) and VOLUME DOWN/UP buttons to adjust it. The adjustable balance ranges from –31 to +31, and other adjustments from 0 to 63. The SOUND menu display disappears and the TV performs the corresponding operation if there is no operation or any other buttons than CHANNEL DOWN/UP and VOLUME DOWN/UP buttons are pressed within 6 seconds.

TIME Menu

Use the CHANNEL DOWN/UP buttons to move between the various settings (such as the current time, On Channel, On Time, Off Time and Sleep Time) in the TIME menu. You may set On Channel 1/2, On Time 1/2 and Off Time via the Program Table, Use the VOLUME DOWN/UP buttons to adjust the settings (e.g. use the VOLUME DOWN/UP buttons to set the hour from 0-23 or set the minute from 0-59). You may enter one to three effective digits directly with the Channel Number buttons but not with the VOLUME DOWN/UP buttons to set On Channel to any one of 0-235. If the time when the TV is programmed to On Channel 1 or On Channel 2 is up in the Power On mode, the TV will tune to On Channel 1 or On Channel 2. During auto program, Off Time will not be performed and be cleared even if the Off Time is up. If the on times are the same as the off time, the TV will perform last channel number setting and clear other timer settings. In the Standby mode, only when the first On Channel meets the requirement, can On settings are performed. And other settings will be cleared after they meet the requirements.

Function Menu

Use the CHANNEL DOWN/UP buttons to highlight one of adjustments and VOLUME DOWN/UP buttons to adjust it.

- You may select one of the eight languages for on-screen display – English, French, German, Russian, Indonesian, Malayan, Arabic or Farsi.

- Color Temp can be set to one of three modes Normal, Cool or Warm in turn.

- If with Blue Back on, the screen will display a blue background when no signal is received.

- If with PIC Enhance on, the picture will be improved.

- With AVL (auto volume leveling), it automatically stabilizes the audio output signal to a certain level. The mode before power off is default value.

TUNING Menu

In the TUNING menu, you may select any of “Auto Program”, “Search”, “Fine Tune”, “Color”, “Sound”, “Band”, “Skip” and “Program”.

- Color

When “Color” is highlighted, use the VOLUME DOWN/UP buttons to switch the current color system to one of the following items and store it in the related unit of the EEPROM.

AUTO-->PAL--> NTSC-->NTSC443-->SECAM—>AUTO

(Note: NTSC/NTSC443 are option set.)

If auto program begins immediately after you change color system, the TV will auto program always in Auto color system.

- Sound

When “Sound” is highlighted, use the VOLUME DOWN/UP buttons to switch the current sound system to one of the following items and store it in the related unit of the EEPROM.

If auto program begins immediately after you change sound system, the TV will auto program in the adjusted sound system.

D/K    B/G     I     M

When OP＿Sound＿DK is set to 1, D/K sound system is available.

When OP＿Sound＿BG is set to 1, B/G sound system is available.

When OP＿Sound＿M is set to 1, M sound system is available.

When OP＿Sound＿I is set to 1, I sound system is available.

Note: When only one sound system is available, no “Sound” appears in the TUNING menu.

- Band

When “Band” is highlighted, use the VOLUME DOWN/UP buttons to switch the band to one of the three items VHFL, VHFH and UHF.

- Skip On/Off

When “Skip” is highlighted, use the VOLUME DOWN/UP buttons to select Skip On/Off. Press the CHANNEL DOWN/UP buttons and you will find the channel number with Skip On set skipped. But during swap, press the VOLUME DOWN/UP buttons and the channel number will not be skipped.

- Fine Tune

After selecting “Fine Tune”, press the VOLUME DOWN/UP buttons to start fine tuning. Use the VOLUME UP buttons to fine tune upwards (when you press the VOLUME DOWN or UP button once, the TV will decrease or increase the tuning value by one 250ms after the button is released and store the value into a unit in EEPROM corresponding to the current channel number. After the button is released, the TV will access the TUNING menu after the set stable time of 4 seconds is up.) The adjustment bar on the screen changes accordingly as the tuning value increases or decreases. When the tuning value reaches the limitation of a band, the TV will fine tune to the next band. AFT on the fine tuned channel is automatically set to Off.

- Search

Use the VOLUME DOWN or UP button to start search as desired. When the TV searches a channel at end of a band, the TV will automatically search another band. After a channel is searched, search stops and the TV returns the original menu display mode and stores the tuning data, band and Skip Off into a unit in EEPROM corresponding to the current channel number. The color/sound systems are set by users.

Use the MENU button to stop search. The current tuning mode remains unchanged and the TV accesses the TUNING menu. During search, adjustment bar on the screen changes accordingly. After search, the adjustment bar disappears.

- Auto Program

After “Auto Program” in the TUNING menu is selected, auto program begins from CH No.1 and blue back is not available. The searched channels are set to Skip Off with color system set to Auto and sound system set by users. Auto program begins from minimum tuning value at VHF-L to maximum tuning value at UHF. Tuning data, band and SKIP OFF of all the searched channels are stored into the related unit in EEPROM. Press the VOLUME UNP/DOWN buttons so that color system of the auto programmed channels is set by user and sound system is the same as that at the beginning of auto program. In the course of auto program, any buttons other than the following specified do not function and the AUTO OFF function does not work, either.

During auto program, adjustment bar on the screen changes accordingly. In this case, press the MENU button to stop auto program and the TV returns to CH No.1 with the TUNING menu still existing. The menu display disappears 6 seconds after last selection. During auto program, you may also press the STANDBY button to stop auto program and return to the Standby mode. Pressing any other button will not function for the TV. When the TV finishes searching throughout the three bands or 236 positions are occupied, auto program automatically stops and the left channel numbers are set to Skip On with other parameters keeping the same.

- Program

When “Program” is highlighted, use the VOLUME DOWN/UP buttons to change the current channel number. In this case, you may call up the skipped channel even if it is set to Skip On, different from the functions of CHANNEL DOWN/UP buttons.

When you enter two-digit channel number with 0-9 buttons, the TV will automatically swap the tuning value of the current channel number with that of the entered channel number.

SWAP Menu

Call up the SWAP menu. Use the CHANNEL DOWN/UP buttons to select one of “Channel A”, “ Channel B” and “Swap”. Use the Channel Number buttons to enter a channel number. In this case, pressing the VOLUME DOWN/UP buttons can not change channel numbers. Use the SWAP button to swap the channel numbers.

BIORHYTHM Menu

Use the CHANNEL DOWN/UP buttons to select “Birthday”or “Today” and VOLUME DOWN/UP buttons to change the settings. Then the biorhythm at that day (such as Energy, Intelligence and Spirit) will be displayed as icon on the screen.

HOTEL Menu

Press the MENU button repeatedly to access the HOTEL menu. Only after entering the correct password using the Channel Number button, can you adjust the following items: On Position, Max Volume, Tuning Lock, Program Lock, AV Lock and New Password. (Note: For some models, the menu is not available.)

Note:

You may change the effective digits of the channel numbers with the DIGIT SELECT button (If effective digit is displayed as -, directly enter 1; if displayed as ---, you may enter 001.

- TV/AV button

Press the TV/AV button to cyclically select signal source among TV, AV (AV1 if with two-way AV input options; or RGB if with RGB signal input to the SCART jack), S-VIDEO (if option), DVD (if option) and AV2. In the AV (or RGB) mode, use the --- button to select one-digit, two-digit or three-digit and the 0-9 buttons to enter an effective channel number. At this time, the TV returns to the related antenna broadcast reception. If with AV or RGB signal input to the SCART jack each time power-on, the TV directly enters the AV or RGB mode. Otherwise, it directly enters the TV mode (if with this option)

    - To Inspect Functions of Buttons on Remote Control

- STANDBY button

Press MAIN POWER SWITCH button and then STANDBY button to turn on the TV. The TV enters the normal reception mode. Press the STANDBY button again to enter the STANDBY mode.

- MUTE button

During normal play, press the MUTE button to mute the sound and display MUTE (or mute symbol) on the screen. Press the button again to restore sound. In the MUTE mode, press the VOLUME UP button to increase volume from the position where mute begins and the VOLUME DOWN button to decrease volume from the position where mute begins.

- DIGIT SELECT button

Press the --- button repeatedly to display “-”, “--” and “---” in turn.

- AV/TV button

The same as the TV/AV button on the TV.

- MENU, CHANNEL DOWN/UP and VOLUME DOWN/UP buttons

The same as the buttons on the TV

- DISPLAY button

With any display on the screen (except display during auto program), press the DISPLAY button to turn off the display.

Without display on the screen, press the DISPLAY button to display information about the current program, system and clock.

- RECALL button

Press the RECALL button and the TV will directly skip back to the previous channel.

- BIORHYTHM button

Press the button to call up the BIORHYTHM menu. Use the CHANNEL DOWN/UP buttons to select “Birthday”.or “Today”and VOLUME DOWN/UP buttons to change the settings.

- PICTURE button

Press the PICTURE button to select one of the following modes: Nature, Vivid, User and Mild.

- SOUND button

Press the SOUND button to select one of the following four modes (not available for the TVs without audio processing): User, News, Theatre and Music.

- TEXT button

Receive CH-Z1 pattern. Press the TEXT button to enter the TEXT mode. No picture appears on the screen with black background. “100” appears on upper left of the screen and text message is displayed. Press the button again to quit from the TEXT mode and resume broadcasting view.

- INDEX button

In the TEXT mode, press the button to access the INDEX page (100).

- HOLD button

In the TEXT mode, press the button to hold (not refresh) the current text page display

- REVEAL button

In the TEXT mode, press the button to check up the concealed information on the current page. After displayed for a time, the concealed information disappears.

- ZOOM button

In the TEXT mode, press the button to split a page into two. Size of characters on each part is doubled. Press the button once to display the first half page content and press the button twice to display the second half page content. Press the button thirdly to return to normal display mode.

- Text Color buttons (Red, Green, Yellow and Cyan)

In the TEXT mode, press any of the buttons to display a text page corresponding to the color.

- CANCEL button

Press the button to access the CANCEL mode and display picture on the screen. If entering a page number at this time, the page number will appear on the screen when the corresponding page number is found.

- MIX button

Press the button to enter the MIX mode.

- SUBCODE button

In the TV mode, press the button to display text time. In the TEXT mode, press the button to display “****” on the upper left corner of the screen if without appendix for the current page; or display “1234” if with appendix. In this case, enter the corresponding page number to display the appendix and picture at the same time.

- HELP button

Press the button to display the help information on the upper right corner of the screen.

- AUTO button

Press and hold the button for two seconds and the TV start the intelligent operation as follows:

  - Check if the TV is in the AV mode. If so, enter the TV to the TV mode firstly. If not, the TV operates as follows.

  - displays the current channel number, color/sound systems, etc. on the screen for 2 seconds. (the same as the display when pressing the DISPLAY button).

  - switches the current language to the preset language, refreshes and holds the FUNCTION menu for 2 seconds.

  - turns off Mode 4 display and display the current volume color bar. If the volume level doesn’t reach 20, approach it to 20 (volume bar is always displayed during adjustment), otherwise access the Mode 6 display directly (if the volume is locked to less than 20 by the hotel lock, take the volume as standard.)

  - turns off Mode 5 display and displays the current picture mode for 2 seconds.

  - switches the current picture mode to Nature and hold it for 2 seconds.

  - turns off Mode 7 display and displays the current Color system in the Tuning menu for 2 seconds.

  - switches the current color system to “Auto” and displays the Tuning menu for 2 seconds.

  - Start auto program (the same as auto program operation via the Tuning menu).

As the intelligent operation is designed for users not familiar with TV functions, the button should be pressed and held for 2 seconds. If with HOTEL options, pressing the button should skip the locked item.

(Note: This function works only in the TV mode.)

Notes:

  - Inspection of the remote function differs depending on the remote controls used.

  - It’s not necessary to check the teletext function for the TVs without this function.

6. RF AGC Adjustment and Contrast Inspection

6.1 Equipment and Signal to Be Required

- Factory RC

- Oscilloscope

- Signal amplitude of 63dBμV

5.2 Adjustment Method

- Receive CH-3 color bar pattern with signal level of 63dBμV. Press the TOP button on the factory RC to adjust until noise dots just appear on the screen. Press the –1 button to adjust AGC. Check and make sure that the picture and sound should be normal with 90dBμV level received and signal-to-noise ratio O.K with 40dB level received. Set AGC from 10 to 28 to avoid different results caused by different alignment personnel.

- Connect probe of the oscilloscope to Kb of the CRT RGB PCB and set the brightness, contrast and color to 32, 63 and 0 respectively. Check the contrast value and make sure it is 75±5V as shown below.

75±5V Vpp

—— —— —— —— —— —— —— —— —— —

Fig. 1

- Check signals from other channels and ensure that the picture and sound are normal.

7. Picture Geometric Distortion Correction

- Receive CH-5 PAL circle pattern. Press the V-HALF button on the factory RC and the TV is displayed as a half screen. Adjust it until the first half part of picture is displayed as a whole. Press the V-HALF button again to return to normal vertical scan.

- Press the V-SHIFT button on the factory RC and adjust the picture until vertical center of the picture coincides with vertical center of the CRT and center of the picture is within the range of ±2mm away from vertical center of the CRT.

- Receive CH-6 crosshatch pattern. Press the SC button on the factory RC and adjust the picture until crosshatch in the lower and upper picture coincides with that on center of the CRT.

- Receive CH-5 PAL circle pattern. Press the V-AMP button on the factory RC and adjust vertical amplitude until the video vertical amplitude is within the range of 2mm away from lower or upper chessboard disappearing dots. Over 2mm is allowed if vertical reappearance ratio is ensured to be not less than 90% and regular circle can be available.

- Press the H-SHIFT button on the factory RC and adjust the picture until horizontal center of the picture coincides with horizontal center of the CRT.

- Adjust the horizontal amplitude inductor L506. Allow horizontal center of the picture is within the range of ±3mm away from center of the CRT.

- Receive CH-13 red full-field pattern. Check and ensure video reappearance ratio to be not less than 90%. If not, repeat the steps mentioned above.

- Receive CH-10 N circle pattern. Adjust the picture to the optimum mode with the V-SHIFT, V-AMP and H-SHIFT buttons.

Note:

Only in the above way can you adjust video horizontal/vertical position.

Fig. 9 Illustration for Horizontal/Vertical Position

Fig. 2

8. Character Position Adjustment

Receive PAL and NTSC circle patterns respectively. Press CHA POS button to adjust 5V0F/6V0F data so that top of characters “0-3F” within 5V0F/6V0F data keep level with the lower black frame of the logo and the primary menu locates on more upper left of the screen, thus ensuring whole page of teletext information displayed (for TVs with NTSC signals reception only).

9. Vertical Sync Inspection

9.1 Inspection Equipment

47Hz signal generator

9.2 Inspection Method

- Input 47Hz signal so that a stable horizontal line is obtained.

- When changing channels, the picture should be stable without up and down rollover.

10. Inspection for Picture and Color

- Turn on the TV and receive CH-5 PAL circle pattern.

- Check picture sharpness dependent on definition of 3.8MHz in multiburst pattern.

- Check high/low frequency reflection after observing black line with white background on top of the circle pattern and white line with black background on bottom of the circle pattern.

- Check if the colors of the circle pattern arrange in the order of white, yellow, cyan, green, purple, red, blue and black.

- Check time delay difference between the brightness channel and color channel depending on distinctness of the vertical boundary of yellow/red step signals on bottom of the circle pattern or stagger boundary.

- Check if phase and PAL delay are normal after inspecting color signals near the circle pattern. If phase of reference sub carrier of the sync demodulator in the TV is correct and delay demodulation and PAL switch are normal, the vertical bar on the left/right of circle close to the frame should have no color, based on which you may check amplitude proportion of primary color signals sent to the CRT’s three cathodes for normality. If amplitude error occurs in delay and direct signals of the delay demodulator, line crawl exists on the parts mentioned-above. If phase error occurs, line crawl exists on the second left vertical line and second right vertical line of the circle pattern, which needs to be serviced. In normal mode, top and bottom of the second left vertical line turn cyan and red respectively; top and bottom of the second right vertical line turn yellow and blue respectively; top and bottom of the third left/right vertical line turn blue and yellow respectively.

- Receive SECAM color bar pattern. Check if the color bars arrange in the order of white, yellow, cyan, green, purple, red, blue and black and color is normal (for TVs with SECAM signals reception only).

- Receive NTSC 3.58MHz color bar pattern. Check and make sure the color bars arrange in the order of white, yellow, cyan, green, purple, red and black and color is normal (for TVs with NTSC signals reception only).

11. Inspection for Purity and Convergence

11.1 Purity Inspection

- Receive white full-field pattern (or red full-field pattern).

- Degauss the CRT.

- Check purity with white (monochromatic) signal in magnetic-free equipment and ensure to come up to the following standard. If not, readjust it again. Observe with a magnifying glass if necessary.

Section A: Purity offset is not allowed.

Note: Section A is a circle with effective screen height as its diameter.

Section B: 1/4 purity offset is allowed.

B

A

Fig. 8 Sectional Diagram of Purity Inspection

11.2 Convergence Inspection

- Receive crosshatch pattern.

- Decrease the brightness and contrast properly to check convergence.

- Check if the convergence meets the following specifications.

- Convergence difference dependent on time in Section A should be less than 0.5mm.

- Within supply voltage of 90-250V AC, convergence difference in Section A should be less than 0.5mm.

B

B              B

B

C

C

C

C

A

A= 20mm×30mm

C= 60mm×60mm

Dyeing line width in Section A should be below 0.5mm.

Dyeing line width in Section B should be below 1mm

Dyeing line width in Section C should be below 1.8mm

Fig. 9 Sectional Diagram of Convergence Inspection

12. Inspection for RF NTSC/PAL/SECAM, Audio and AV Input/Output

12.1 Inspection Equipment and Signal

- Oscilloscope

- Factory RC

- 70dBµV standard multi audio system color TV signal source

- Color video signal source PAL

- Color video signal source NTSC

- Color video signal source SECAM

- RGB video signal source (for TVs with the function only)

- YCrCb component signal source (for TVs with the function only)

- Y/C signal source (for TVs with the function only)

- Audio signal source

12.2 Inspection Method

12.2.1 Inspection for RF NTSC/SECAM and Audio

- Receive standard RF color TV signals in M, B/G, I and D/K respectively from signal source of 50KHz FM modulation 1KHz deviation. Press the D/K, B/G, I or M button to select a correct audio system until speakers output normal noise-free sound.

- Receive RF signals in PAL, NTSC and SECAM respectively. Adjust the TV until normal picture and sound are obtained under the corresponding color system. In NTSC, tint changes accordingly as adjusting tint control. Color bar sequence should be normal when tint is set to optimum (visual estimation). In SECAM, color sequence should be normal.

Notes:

  - For TVs without NTSC or SECAM signals reception, it’s not necessary to inspect the related system.

  - For TVs without I, D/K or M audio signals reception, it’s not necessary to inspect the related audio system.

12.2.2 AV Input Inspection

- Set the TV to the AV mode. Send 1Vpp (Loading: 75Ω) PAL, NTSC and SECAM video signals and 1Vpp sine wave audio signals into the VIDEO/AUDIO IN terminals on the front panel respectively. Check and make sure that the picture, sound and color in every system are normal. Tint adjustment and audio effect processing should also be correct.

- Set the TV to the AV mode. Send 1Vpp (Loading: 75Ω) PAL, NTSC and SECAM video signals and 1Vpp sine wave audio signals into the SCART jack on the TV respectively. Check and make sure that the picture, sound and color in every system are normal. Tint adjustment and audio effect processing should also be correct.

- Input 0.75Vpp (Loading: 75Ω) RGB signals, sync signals and audio L/R signals. Set the TV to the RGB mode. Check and make sure the picture is sharp and normal without Y/C cross talk, both of which should be better than those in a, b. Ensure sound to be normal.

- Switch the TV to S-VHS mode and send Y/C signals to the S-VIDEO IN terminals. Check and make sure the picture is sharp and normal and no distinct Y/C cross talk exists.

- Switch the TV to DVD mode and send YCrCb signals. Check and make sure the picture is sharp and normal and no distinct Y/C cross talk exists.

- During the inspection, no distinct interference between AV IN and TV IN should be found.

Notes:

  - It’s not necessary to check SECAM signal input if TVs couldn’t receive SECAM signal.

  - It’s not necessary to perform the inspection in Step b for TVs without SECAM signal reception.

  - It’s not necessary to perform the inspection in Step c for TVs without RGB signal reception.

  - It’s not necessary to inspect audio effect for TVs without AV stereo.

  - It’s not necessary to perform the inspection in Step d for TVs without S-VIDEO IN terminals.

  - It’s not necessary to perform the inspection in Step e for TVs without Y, Cr, and Cb terminals.

  - For some TVs, do not use the AV1 IN, S-VIDEO IN, DVD IN and SCART IN terminals at the same time.

12.2.3 AV Output Inspection

- Power on the TV to receive Z-37 pattern.

- Measure audio output to be 1±0.2Vpp with the oscilloscope.

- Measure video output to be 1±0.2Vpp (Loading: 75Ω) with the oscilloscope.

Note: Only in the TV mode can AV output be normal. So inspect AV output only in the TV mode.

13.Volume Inspection

Under the normal audio system, set treble and bass to 32 and balance to 0 respectively. Adjust volume control and measure voltage on two ends of the speaker to be not less than 10Vpp using the oscilloscope with audio FM modulation 50KHz deviation reception (such as white full field signal).

Ⅳ FINAL INSPECTION AND OUTGOING OPTION

Check the following items after final assembly of the TV.

1. Inspection for Geometric Distortion

  - Inspection Signal

CH-5 circle pattern with 75dBµV signal level

  - Inspection Method

Receive CH-5 circle pattern. Check and make sure the picture geometric distortion does not exceed 3%, frame/horizontal reappearance ratio not less than 90% and the picture geometric center coincides with center of the CRT.

2. Inspection for Buttons on the TV and Character Position

- Press the buttons on the TV and ensure they work normally.

- Press the VOL+ button to display volume adjustment character on the screen. Ensure the same distance between the adjustment character and two ends of the screen. 2.5cm between the character and center is allowed.

3. Inspection for Mute and Blue Back under No Signal Condition

Receive CH-5 circle pattern and make sure the sound is normal. Pull off the antenna cable and no sound should be heard from the speakers. Select blue back on or off and even blue back should appear or disappear on the screen. Under no signal condition, the TV will automatically shift to the Standby mode in 15 min.

4. Confirmation for Supply Voltage Range

The TV should work normally (e.g. picture and sound are normal) when the supply voltage ranges from 90V AC to 250V AC.

5. Safety Inspection

5.1 Inspection for Insulation and Voltage-resistant

Test safety of all naked metal of the TV. Supply high voltage of 3000V AC, 50Hz (limit current of 10mA) to the TV after ensuring that test rod contacts with tested point well. Test every point for 3 seconds and ensure no arcing and sparking.

5.2 Requirements for Insulation Resistance

Measure resistance between naked metal of the TV and feed end of the power cord to be infinity with the DC-500 high resistance meter and insulation resistance between the naked metal and degaussing coil over 20MΩ.

6. Inspection for Weak Signals and Remote Functions

After safety inspection, receive CH-5 circle pattern. Make sure that signal-to-noise ratio is O.K when signal strength is 40dBµV. The sound and picture should be normal when signal strength is 90dBµV. The TV should operate normally by using the remote control.

7. Inspection for Back Cover, Front Panel and Structure

- Check the front panel and screen for being clean or having scratches.

- Check every label for correctness.

- Check every button’s position.

8. Outgoing Setting

- Press the OUTGOING button on the factory RC.

Perform outgoing setting as follows.

    - Set the picture mode to Vivid.

    - Set the sharpness to 32.

    - Set the bass to 32.

    - Set the treble to 32.

    - Set the balance to 32.

    - Set the tint to 32.

    - Set the volume to 30.

    - Set the color system to AUTO.

    - Set the sound system to B/G.

    - Set the language for OSD to English.

    - Set AVL to OFF.

    - Set the color temp to Standard.

    - Set the blue back to ON.

- Press the M button on the factory RC to quit the M-MODE.

Notes:

  - For TVs without some functions, it’s not necessary to implement the related outgoing setting.

  - If some items can’t be set by the OUTGOING button, manual set them by the factory RC.

9. Accessories

Operation Manual          1

Remote Control            1

AAA Alkaline Batteries      2

APPENDIXⅠ TUNING MENU AND DATA OPTIONS

Data Options in the S Mode

| Data in M Mode | Adjustment | Description | Remarks |
| --- | --- | --- | --- |
| 5PAR/6PAR | 1F | Parallelogram correction (for large-screen only) |  |
| 5BOW/6BOW | 1F | Curve correction (for large-screen only) |  |
| 5HSH/6HSH | Set to the optimal mode | Horizontal center When vertical frequency of 50Hz is identified, “5HSH” is displayed；when vertical frequency of 60Hz is identified, “6HSH” is displayed. | * |
| 5EWW/6EWW | 1F | East-West correction (for large-screen only) |  |
| 5EWP/6EWP | 1F | East-West parabola correction (for large-screen only) |  |
| 5UCR/6UCR | 1F | Top parabola correction (for large-screen only) |  |
| 5LCR/6LCR | 1F | Bottom parabola correction (for large-screen only) |  |
| 5EWT/6EWT | 1F | Trapezoidal correction (for large-screen only) |  |
| 5VSL/6VSL | 1F | Half vertical output | * |
| 5VAM/6VAM | 1F | Vertical amplitude When vertical frequency of 50Hz is identified, “5VSM” is displayed；when vertical frequency of 60Hz is identified, “6VSM” is displayed. | * |
| 5SCL/6SCL | Set to the optimal mode | S correction | * |
| 5VSH/6VSH | Set to the optimal mode | Vertical center When vertical frequency of 50Hz is identified, “5VSH” is displayed；when vertical frequency of 60Hz is identified, “6VSH” is displayed. | * |
| 5VOF/6VOF | Set to the optimal mode | OSD vertical center | * |
| VX | 19 | Vertical zoom (for large-screen only) |  |
| RED | 20 | Red gun cutoff voltage | * |
| GRN | 20 | Green gun cutoff voltage | * |
| WPR | 1F | Red gun drive voltage | * |
| WPG | 1F | Green gun drive voltage | * |
| WPB | 1F | Blue gun drive voltage | * |
| YDFP/ YDFN | 07 | Brightness delay time |  |
| TOP | 1C | UOC AGC | * |
| VOL | 22 | UOC audio output amplitude | * |
| IFFS | 03（02） | PIF (03-38MHz) 02-38.9MHz |  |
| HDOL | 07 | Cathode drive level |  |
| AGC | 03 | IF ACG speed |  |
| VG2B | 30 | VG2 brightness | * |
| SBRI | 1F | Sub brightness |  |
| MBRI | 2F | Max. brightness |  |
| SCON | 20 | Sub contrast |  |
| MCON | 39 | Max. contrast |  |
| SCOL | 32 | Sub color |  |
| OP1 |  | Option set byte 1 |  |
| OP2 |  | Option set byte 2 |  |
| OP3 |  | Option set byte 3 |  |
| OP4 |  | Option set byte 4 |  |
| OP5 |  | Option set byte 5 |  |
| OP6 |  | Option set byte 6 |  |
| INIT |  | EEPROM initialization |  |
| VG2 |  | Adjusting screen voltage with VG2 |  |
| VSD |  | Horizontal bright line |  |
| LOGO |  | User logo write-in (valid when OP-USER-LOGO is 1) |  |
| STS0/1/2 |  | System status byte |  |

Notes:

The data marked with “*” can be adjustable in the M-MODE. Caution should be exercised when adjusting other items.

Function Option Bit Setting

|  | Bit | Item | Description | Default |
| --- | --- | --- | --- | --- |
| OP1 | 0 | OP_HOTEL | HOTEL mode | 1 |
|  | 1 | OP_COMB_FILTER |  | 0 |
|  | 2 | OP_NTSC |  | 1 |
|  | 3 | OP_AV2 | Two sets of AV terminals: 1-With AV2; 0-Without AV2 | 1A1-1 1A/1A2-0 |
|  | 4 | OP-SVHS | S-Video terminal | 1A1-1 1A/1A2-0 |
|  | 5 | OP-DVD | DVD | 1A1-1 1A/1A2-0 |
|  | 6 | OP-RGB | RGB: 1- With RGB 0-No | 1A1-0 1A/1A2-1 |
|  | 7 | OP-OSO | Switch off in vertical overscan | 0 |
| OP2 | 0 | OP-AVL | AVL | 1 |
|  | 1 | OP_AUTO_SOUND | Auto sound system test | 1 |
|  | 2 | OP_NOT_1 | National Option Set | 0 |
|  | 3 | OP_NOT_2 |  | 0 |
|  | 4 | OP_NOT_3 |  | 0 |
|  | 5 | OP_FUNC1 | User logo | 0 |
|  | 6 | OP_FUNC2 | ON BACKGROUND | 0 |
|  | 7 | OP_FSL | Slicing level for vertical sync | 0 |
| OP3 | 0 | OP_ENGLISH | English | 1 |
|  | 1 | OP_FARSI | Farsi | 1 |
|  | 2 | OP_ARABIC | Arabic | 1 |
|  | 3 | OP_RUSSIAN | Russian | 1 |
|  | 4 | OP_FRENCH | French | 1 |
|  | 5 | OP_GERMAN | German | 1 |
|  | 6 | OP_INDIA | Indonesian | 1 |
|  | 7 | OP_MALAYSIA | Malay | 1 |
| OP4 | 0 | OP_FMWS | Window selection of sound pll: small/large window | 0 |
|  | 1 | OP_DIRECT_SWITCH_ON | Memory power-on | 1 |
|  | 2 | OP_HCO | EHT tracking mode | 0 |
|  | 3 | OP_ LOGO | User logo display：1-Displayed without signal reception；0-No | 1 |
|  | 4 | OP_SOUND_DK | Sound system option set | 1 |
|  | 5 | OP_SOUND_BG |  | 0 |
|  | 6 | OP_SOUND_I |  | 1 |
|  | 7 | OP_SOUND_M |  | 1 |
| OP5 | 0 | OP_TUNER | Tuner: 1 Philips→Tuner 0→Panasonic Tuner | 0 |
|  | 1 | OP_AUTO_LANG0 | Auto language options | 1 |
|  | 2 | OP_AUTO_LANG1 |  | 0 |
|  | 3 | OP_AUTO_LANG2 |  | 0 |
|  | 4 | OP_FORF | Field frequency options | 0 |
|  | 5 | OP_FORS |  | 0 |
|  | 6 | OP_AVON | If AV off, then AV on | 1 |
|  | 7 | OP_ONPOSITION | With HOTEL mode preset, on position is fixed to POS1. | 0 |
| OP6 | 0 | OP_AUTOTEST | Auto test when power-on | 0 |
|  | 1 | OP_PSNS | Sensitivity | 0 |
|  | 2 | OP_BSCREEN | Black screen when changing channels: 1-yes；0-No | 1 |
|  | 3 | OP_XDT | SECAM selection | 1 |
|  | 4 | OP_DFL | Disable flash protection | 0 |
|  | 5 | OP_SIF | External input for sound IF circuit | 1 |
|  | 6 | OP_EXT_SIF0 | Sound system options for external circuit | 1 |
|  | 7 | OP_EXT_SIF1 |  | 0 |

Note:

The data sheet may differ dependent on different models and different CRTs. It’s necessary to check if data options meet the requirement of functions of TVs in production.

TAB Data Inspection Table

TAB

| STS0： | POR | IFI | LOCK | SC | CD3 | CD2 | CD1 | CD0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STS1： | XPR | NDF | FSL | IVW | WBC | HBC | BCF | IN2 |
| STS2： | SUP | X | X | QSS | AFA | AFB | FMW | FML |

Note:

The above data is only for reference. Actual data may differ dependent on different models and different modes.

APPENDIXⅡ EEPROM PRE-WRITE-IN (EEPROM MASTER SLICE)

1. EEPROM data options include:

- functions and data options;

- geometric data adjustment (refer to the Test & Alignment Procedures);

- tuning data:

  - in the USER mode, searching all receivable channels and setting to correct sound system by semi auto program;

  - fine tuning properly frequency of each searched channel to lower level (to 2MHz or so)

2. Set the related data in outgoing setting ways.

APPENDIXⅢ MANUFACTURE MODE

The MANUFACTURE MODE is designed for use in test & alignment of production line. In this mode, only adjust the data causing change in the whole performance due to discrete data of the components. The items to be checked should also be adjusted to the greatest extent during manufacturing.

To adjust the data, a specific factory RC is required. Remote control bit is set to RC-5 and system bit to 0x1F in the MANUFACTURE MODE. To access or quit the MANUFACTURE MODE, use the command bit 0x41 on the factory RC.

| Serial No. | Item | Button | Button Bit | Function |
| --- | --- | --- | --- | --- |
| 1 | AGC | AGC | 1 | RF AGC adjustment |
| 2 | M_HBOR | RC | 2 | Dark balance R gun |
| 3 | M_HBOG | GC | 3 | Dark balance G gun |
| 4 | M_R_ADJUST | R | 4 | R DRV |
| 5 | M_G_ADJUST | G | 5 | G DRV |
| 6 | M_B_ADJUST | B | 6 | B DRV |
| 7 | M_SUPER_WOOFER | WOOFER | 7 | White balance switch |
| 8 | M_BASS_ADJUST | BASS | 8 | Bass |
| 9 | M_TREBLE_ADJUST | TREBLE | 9 | Treble |
| 10 | M_BALANCE_ADJUST | BALANCE | 10 | Balance |
| 11 | M_POWER | POWER | 11 | Data sheet |
| 12 | M_STEP_MAX | MAX | 12 | Adjust data to Max. |
| 13 | M_STEP_MID | MID | 13 | Adjust data to middle |
| 14 | M_STEP_MIN | MIN | 14 | Adjust data to Min. |
| 15 | M_STEP_DOWN_ONE | -1 | 15 | Adjust data-1 |
| 16 | M_STEP_UP_ONE | +1 | 16 | Adjust data +1 |
| 17 | M_STEP_DOWN_FIVE | -5 | 17 | Adjust data -5 |
| 18 | M_STEP_UP_FIVE | +5 | 18 | Adjust data +5 |
| 19 | M_BRIGHT_ADJUST | BRIGHT | 19 | Brightness adjustment |
| 20 | M_COLOR_ADJUST | COLOR | 20 | Color adjustment |
| 21 | M_CONTRAST_ADJUST | CONTRAST | 21 | Contrast adjustment |
| 22 | M_VER_SLOPE | VS | 22 | Half vertical output |
| 23 | M_VER_AMP | VA | 23 | Vertical amplitude |
| 24 | M_VER_SHIFT | VSH | 24 | Picture center |
| 25 | M_S_CORRECT | SC | 25 | S correction |
| 27 | M_VER_VSD | VSD | 27 | Vertical output |
| 28 | M_POS_DOWN | P- | 28 | Channel down |
| 29 | M_POS_UP | P+ | 29 | Channel up |
| 31 | M_HOR_SHIFT | HSH | 31 | Horizontal center |
| 38 | M_SUB_BRIGHT | Brightness 50 | 38 | Sub brightness |
| 39 | M_SUB_CONTRAST | Contrast 50 | 39 | Sub contrast |
| 40 | M_VER_CHR_POS | OV | 40 | Vertical position of character |
| 41 | M_ENTER | M | 41 | M-MODE enter/quit |
| 42 | M_AUTO_SEARCH | AUTO | 42 | Auto program |
| 43 | M_UP_SEARCH | SERACH | 43 | Search |
| 44 | M_NV_INIT | INIT | 44 | EEPROM initialization |
| 45 | M_END_SET | END | 45 | Outgoing setting |
| 46 | M_COLOR_SYS | COLOR SYSTEM | 46 | Color system switchover |
| 47 | M_SOUND_SYS | SOUND SYSTEM | 47 | D/K, B/G, I, M switchover |
| 48 | M_AV_CHANGE | AV | 48 | AV |

In the MANUFACTURE MODE, red “M” appears on the top center of the screen and the current adjustment and its data appear below “M”. To avoid data lost due to sudden power failure during manufacturing, the result renewed each time should be stored in the EEPROM immediately. The MANUFACTURE mode should also be stored as 1BIT into the EEPROM. If the BIT is detected to be 1 after power on, the TV will enter the MANUFACTURE MODE immediately.

M

AGC       12

Plane Figure of Factory RC in the M Mode

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | STS | COLOR SYS | SOUND SYS | RC | GC | R | G | B |
| 2 | AGC | AUTO | SEARCH | M | SEARCH | END | POS- | POS+ |
| 3 | SBRI | SCON | TC | OV | TINT | SHARP | -1 | +1 |
| 4 | HSH |  |  |  |  | MIN | MID | MAX |
| 5 | VS | VG2 | VSH | VSD | VA | SC | -5 | +5 |
| 6 | AV | COLOR | BRIGHT | CONTRAST | BASS | TREBLE | BALANCE | BUS OPEN |