LIST OF MODIFICATION BY UOC VERSION (N1 N2)

| MODEL | UOC Version | N1 S-Version | N2 H-Version | N2 I-Version | Process | Remark |
| --- | --- | --- | --- | --- | --- | --- |
|  | Software version | Ver.1 | Ver.2(OTP) | Ver.2(Masked) |  |  |
|  | M.P. | Up to now | From December 2001 | From March 2001 |  |  |
| TU ; TI | C590; C591 | 100V CH 30pF (2%) | 50V CH 27pF (5%) |  | Radial | Crystal cap. change |
| ALL | C502 | 50V RSS 22uF | 50V RSS 4.7uF |  | Radial | Sound decoupling change |
|  | R573 | 1/6W 2,7kOhm |  | 1/6W 3,9kOhm | axial | Loop filter change |
|  | C570 | 50V B 4700pF |  | 50V B 1200pF | Radial |  |
|  | C571 | 50V B 820pF |  | 50V B 330pF | Radial |  |
|  | C500 | 50V RSS 0,47uF | DELETE |  | Radial | External reset circuit delete |
|  | C501 | 25V RSS 10uF | DELETE |  | Radial |  |
|  | Q510; Q511 | 2SA1980Y | DELETE |  | Radial |  |
|  | D591 | UZ-2,4B | DELETE |  | Axial |  |
|  | R591 | 1/6W 100Ohm | DELETE |  | Axial |  |
|  | R592,593,594 | 1/6W 10kOhm | DELETE |  | Axial |  |
|  | R595 | 1/6W 47kOhm | DELETE |  | Axial |  |
|  | D511 | TZX3V9B | JUMPER |  | Axial |  |

FOR CP-185 EXCEPT PANASONIC