| Sharp | 37VT24H | Slack tape on eject | Idler not coming across in unload mode - change idler or retension idler spring |
| --- | --- | --- | --- |
| Sharp | VC108 | No take up as idler wheel not engaging | mode sw |
| Sharp | VC483 | Int stops in play esp towards end of tape | Clean take up reel sensor mirror |
| Sharp | VC484HM | No or distorted sound on right hand channel | Dry joints IC702 pins 2, 3, 15, 16 |
| Sharp | VC583 | incorrect or no functions | Can be caused by faulty mode switch or cog jumping |
| Sharp | VC583 | Very Int No playback / record or E-E | Dry joints at plug socket CC on Video chroma PCB |
| Sharp | VC750 | TUNING DRIFT | C1411 LEAKY |
| Sharp | VC750HM | Int chews tape/no tape shuffle when stopped | replaced mode sw & belts/idler |
| Sharp | VC780HM | Loading arms not fully loading | Reciprocating spring in loading gear broken |
| Sharp | VCA100 | Goes in to Fwd Search when FFwd pressed | mode sw |
| Sharp | VCA100 | Int refuses to play/rec (symbol shows) | Poor rivetting between pin 10 IC801 to AE4 |
| Sharp | VCA105 | Capstan motor fast | Capst control chip on motor pcb - means new motor ass'y |
| Sharp | VCA105 | Clicking noise from loading block | Worm gear teeth damaged |
| Sharp | VCA105 | Machine would power on and initialise, then shut down with no functions and no display | Replace Timer IC5001 |
| Sharp | VCA105 | No sigs in e-e mode | IC951 33v reg s/c |
| Sharp | VCA105 | Shuts down aft tape inserted & powered up | Timer IC5001 (remove all plugs from front timer pcb to check - should then play pre rec with tab removed) |
| Sharp | VCA105 | Tape damage - Loading arms not fully returning | Timing between sector gear I tooth out - replace damaged sector gear & back tension post lever/gear (Possible root cause) |
| Sharp | VCA105 | Tape jams on eject | Modified latching bar (item 319) & clutch operating lever (item 321) on housing |
| Sharp | VCA105 etc | Back tension lever is stuck on wrong side of the take up guide | Excessive wear of the tension adjusting lever |
| Sharp | VCA105 etc | Capstan motor will not rotate | C6 leaked causing open circuit print clean and repair track and Replace C6 (10UF ) 25V |
| Sharp | VCA105 etc | Chroma processor IC501 improvement | IC501 replaced (TA8644N) |
| Sharp | VCA105 etc | Clock loses time | Clock ref across pins 30, 31 of IC5001 changes freq |
| Sharp | VCA105 etc | Intermittent tape loading or unloading intermittent no eject Half load lever moving irregularly | Replace cam switch Pt No QSW-R0026GEZZ |
| Sharp | VCA105 etc | Slow rewind | Brake torque take up reel is to high due to contamination of the slow brake lever |
| Sharp | VCA105 etc | VCR will not fully eject: or will not accept a tape: | Replace cassette switch and sensor PWB with improved Pt No DUNTK3194HE00 |
| Sharp | VCA105 etc | When pressing STOP or EJECT while in play or record mode: causes the VCR to power down | Tension release lever (item 52) flexes during unlacing causing a delay in the timing of the tension arm: the delay causes the arm to jam the supply pole base (76) which upsets the unload sequence, resulting in IC801 powering down. |
| Sharp | VCA111 | Drum speed excessive | Check FG pulses at motor pcb Printed FG coil - if low replace motor pcb (available separately) |
| Sharp | VCA111 VCT310 VCT510 | The cass intermittently ejects tape the cass tape gets jammed when record mode is selected tape ejects | Pinch roller assy has become deformed replaced |
| Sharp | VCA111HM VCA615HM | Capstan motor bearing altered after Serial No's starting 503 | New Pt No's Capstan motor RMOTN2019GEZZ Screw XHPSD26P06000 |
| Sharp | VCA140 | Capstan motor starts/stops aft 20 mins | Drive chip hot - replace motor - note vcr ser no for correct type |
| Sharp | VCA140 | Int chews tape | Check movement of ½ load arm |
| Sharp | VCA140 VCA5011 | Capstan Motor stop after approx 10 mins play or rec | Friction between the capstan spindle and bearing |
| Sharp | VCA140HM | int chews tapes/lines/poor sound | half load arm int seized due to hardened grease also replace p roller |
| Sharp | VCA30 | Int Ffwd/Rew - idler gear not engaging | Mode sw |
| Sharp | VCA30 | Poor Pback/E-E, & roll | C4401 47µ 10v |
| Sharp | VCA310 | Accepts cassette then ejects it several times then behaves | Replaced slipping loading belt |
| Sharp | VCA36HM | Carries on in fwd search aft 'Play' pessed with sound | Capstan motor |
| Sharp | VCA39 | After replacement of IC801 or models with serial No higher than 41163191 the RF output on/off switch no longer works | To restore feature Fit Pt No RHIX0867GEZZ |
| Sharp | VCA39 | Int stop | Check drum earth spring is present |
| Sharp | VCA44 etc | Remote functions as if door open even when closed | Disassemble handset and adjust PWB to enable door switch to be centralised in its recess |
| Sharp | VCA48 | During reception (E-E) or in LP mode Sibilance can be detected | Add R630 & change values of C601, C619, C620, R605, R617, R620 |
| Sharp | VCA49 | Low sound/noise bars/poor rew | Excess friction on ACE head - relace head |
| Sharp | VCA50 | Buzz on pback - int | C603 pressing against mechs - reposition |
| Sharp | VCA50 VCA72 | Remote control door difficult to open | Remove sliding door and apply Molykote grease to 2 protruding tabs on the remote door |
| Sharp | VCA50 VCA72 | RF blue mute and menu screen appear grey: | C5909 SMD cap wrong value Replace with 8pF |
| Sharp | VCA5011 | Hum bar in e-e/rec; pback ok | D904 leaky |
| Sharp | VCA5011 | Won't accept tape | Incorrect volts IC803 BA6238AM - D805 s/c (12v zener) |
| Sharp | VCA55 | Dead | R905 in psu |
| Sharp | VCA55 | Stays at Fwd search speed aft Play is pressed | Faulty capstan motor |
| Sharp | VCA55HM | No/slow playback - FFwd/Rew ok | Capstan motor faulty (not stiff bearings) |
| Sharp | VCA615 | Display shows zeros only | Timer chip IC5001 RHIX0581 |
| Sharp | VCA63 | Fails to eject; loads/unloads when stop pressed | Mode sw |
| Sharp | VCA63 | Int dead | C9 dry joint |
| Sharp | VCA63 | Tracking errors, capst speed slow, poor col & when pause pressed goes into FFwd & locks up | No drum PG signal Pin 52 IC801. IC702 no I/p pin 28 from head drum. Head drum's pick up coil faulty - able to reconnect it. |
| Sharp | VCA63HM | Poor FF & Rw | reel brakes and springs and dirty capstan |
| Sharp | VCA63HM | Slack tape on eject | Clean or replace mode sw & check idler |
| Sharp | VCA750HM | Off tune slightly in e-e | OK when 'tune/normal' sw in 'tune' posn. Adj bottom IF coil on IF module |
| Sharp | VCB322 | Won't accept tape | plastic piece that opens the tape flap had come out along with its spring, this sits inside the carriage. Someone had tampered with it! |
| Sharp | VCBS97 | 1. Soft picture when using AI function | 1. Replace R2363 VRD-MN2BE332J |
| Sharp | VCBS97 | E-E mode poor sync/ragged verts/roll | C2527 10µ 16v |
| Sharp | VCBS97 VCR Sat combined | Audio level on the E-E channels are lower than the T.V. channels | Replace R1735 with a 8.2K ohm |
| Sharp | VCF360 | Stops int | Take up reel sensor contaminated with grease |
| Sharp | VCH81 | Int timed recs | Mode sw replaced cured it |
| Sharp | VCH81 | No capst | Electro on motor pcb eaten thro' print |
| Sharp | VCH81 | Tries to load without cassette in | Lighthouse LED s/c |
| Sharp | VCH84 | Crackle & hiss on right chan | bad joint pin 24 TB1204F Nicam chip |
| Sharp | VCH84 | Int loss of mechs | Gear ass'y NGERH1128GEZZ |
| Sharp | VCH841 | Int functs & tape stuck | Capst motor int seizes - lub shaft |
| Sharp | VCH88 | No r/c | IR rec outputting garbage - new Rec req |
| Sharp | VCH89 | Following interruption in mains supply Nicam fails to work | Fit a resistor Pt No VRDRA28E563J between C1788 and 3rd solder joint in from edge of PCB by IC1791 |
| Sharp | VCH93 | Rew int | FFwd/Rew idler hole had enlarged - replace |
| Sharp | VCM20 | Damages top tape edges | Take up torque too high - replace pulley reel |
| Sharp | VCM20 | Int functions | Check dry joints plug/skts AO & OA |
| Sharp | VCM20 | LP symbol perm lit | Micro chip |
| Sharp | VCM20 | Q901 blows intermittently & chopper tfmr buzzes | Dry joints opto coupler IC901 |
| Sharp | VCM201HM | Dead | Chopper tr Q901 2SC4231 s/c, 4R7 2w R904 o/c - replace & add heatsink to Q901 |
| Sharp | VCM21 etc | Capst motor goes in rev or not at all | dry joints on capst motor socket |
| Sharp | VCM21HM | Dead, no display | C907 47µ 400v mains smoother & R904 4R7 2w |
| Sharp | VCM24HM VCM25HM VCM64HM | If clock set in 12Hr mode and record time starts before 12 ( am or pm ) and stop time after 12 vidoe continues recording past stop time . ( This will only happen if numeric keys on remote are used to set timer ) | Change IC801 Pt No RH-IX1205GEN8 |
| Sharp | VCM26 | Child lock removal (pin no n/k) | Press & hold lock key on r/c - lock symbol then displayed, Press & hold st/by on r/c & front of vcr for 3 secs. VCR will shut down & clear child lock |
| Sharp | VCM26 | Childlock removal | Simultaneously press the standby buttons on the front panel and the remote control. |
| Sharp | VCM26 | clock gained 1 min per day | not osc xtal ; one of sm caps in osc never fitted |
| Sharp | VCM26 | Dead & tripping after metal object inserted | "-25v" line missing; R995 10R sm o/c |
| SHARP | VCM26 | INTERMITTANT NO COLOUR | DRY JOINTS ON X501 |
| SHARP | VCM26 | NO COLOUR ON PLAYBACK OR RECORD E-E OK | CHECK C512 FOR S/C |
| Sharp | VCM26 | Patterning all modes & int col | X501 IC201 |
| SHARP | VCM26/27/67 | DEAD, POWER SUPPLY TRIPPING | POSSIBLE R995 SMD O/C SITS ADJACENT TO CHOPPER TX SHARVRSCY1JF100Y |
| SHARP | VCM26HM | NO PLAYBACK VIDEO BUT SOUND OK | CHECK X501 FOR DRY JOINTS |
| Sharp | VCM26HM | Re-alignment of mechs | Connect jumper between TP703 & TP704 before commencing |
| Sharp | VCM27 | Interference on e-e & pback | RF modulator faulty |
| Sharp | VCM27 | No rec/erase sound - bias osc freq & amp too high | Print repaired at C623 at base of bias osc |
| Sharp | VCM27 | Tripping psu | IC901 opto coupler FX0007GE |
| Sharp | VCM271 | Accepts tape then ejects it | Start & end sensors replaced |
| Sharp | VCM271 | Capst int runs in reverse | Dry joints Socket P702 (Capst motor plug) |
| Sharp | VCM271 | Dead - PSU volts low & pulsing | Q921 2SB709 sm leaky |
| SHARP | VCM271 | NO E-E SOUND | REPLACE IF PCB SHARRIFU0672GEZZ |
| Sharp | VCM271 | No erase - bias osc not running | IC701 micro not sw bias on - micro faulty |
| Sharp | VCM271 | Setting RF Modultor | Put vcr into standby, press and hold menu until display shows CH36 (or other ch)...use cursor up key to step up rf channel...press set...then press stop on r/c. |
| Sharp | VCM271 | Won't accept tape | Lighthouse LED |
| SHARP | VCM271HM | DEAD WITH TRIPPING NOISE | CHECK R995 FOR O/C OR HIGH VALUE |
| Sharp | VCM27HM | No playback colour | Xtal X501 on main pcb dry joint |
| Sharp | VCM29 | No E-E pic & no col Pback | X501 Xtal dry joint |
| Sharp | VCM301HM | Int stops in play - drum motor stops | BA6977S IC702 drum drive chip |
| SHARP | VCM311 | NOT ERASING | REPLACE R658 1OHM AND Q651 2SC3203 |
| Sharp | VCM311HM | no erase of old soundtrack - no bias osc | Q651 2SC3203 osc tr leaky, R658 4R7 o/c, Q652 DTC323 |
| Sharp | VCM311HM | No play - load arms only go halfway & return | No drum FG pulse from drum motor IC BA6977S - chip u/s |
| Sharp | VCM311HM | Snowy pback - counter runs fast | Q652 digital DTC323 s/c leaving osc running continuously |
| Sharp | VCM321 | Dead - no display | 2SK2848 power mosfet & IC902 AN8027 brought set to life but no mechs & reverts to st/by. R904 4R7 input buffer res o/c |
| Sharp | VCM321 | Didn't erase prev recs | Q652 DTC323TK sm dig tr (bias sw) |
| Sharp | VCM60 | Dead | R904 4R7 o/c; Q901 2SC4231 s/c - replace & fit heatsink (Pt No PRDAF1065UMFW) or find one in your spares box! |
| Sharp | VCM721 | No take up | Dry joints at plug AC on capst motor |
| SHARP | VCMH26-69HM | DEAD | REPLACE R955 10,OHM RESISTOR SHARVRSCY1JF100J |
| Sharp | VCMH60HM | No rec pic - old rec still present | No bias at FE head due to 8v bias volts missing - Q950 faulty |
| Sharp | VCMH64 | Background hiss on Pback | Dry joint pin 4 plug AU on main pcb |
| Sharp | VCMH64 | int loss of hi-fi sound in rec (1 chan only) | NICAM chip |
| Sharp | VCMH67 | Childlock removal when PIN not known | Press "Childlock" on R/C - when display changes to 4 dashes waiting for correct code press St/by on R/C & front panel together |
| Sharp | VCMH67 | Eject button pushed in | Pt no 27756QA WVE £1.34 |
| Sharp | VCMH67 | Mechs jammed | White plastic leverdouble action (pt no 27864R) WVE attached to pinch roller (pt no 27864M) broken off - supaglued back |
| Sharp | VCMH67 | Tape edge damage | Adj height of 1/2 load arm up 1/2 a turn |
| Sharp | VCMH67 | Using PDC on timed recs | Must have tuner/if on to rec PDC info |
| Sharp | VCMH67 | Won't accept tape | Faulty end sensor (tested ok) |
| SHARP | VCMH67HM | ONLY AUTOTUNE MENU APPEARS | UNIT IS PROBABLY IN CHILD LOCK. TO RELEASE PRESS AND HOLD THE "KEY" BUTTON UNTILL "----" APPEARS IN THE DISPLAY THEN ENTER THE CODE OR PRESS THE STANDBY BUTTONS ON BOTH THE UNIT AND REMOTE AT THE SAME TIME |
| Sharp | VCMH711 | Dead - no display | Q704 sm - 1 leg never soldered |
| Sharp | VCT310 | Hum bar on rec | AT9v has ripple - C905 2200µ |
| Sharp | VCT310HM | Damages tapes on eject . | Check and lightly oil idler shaft , replace mode switch . |
| Sharp | VCT310HM | Jams when tape loading | Worm wheel 19 lost teeth - replaced & main cam/belts |
| Sharp | VCT310HM | No FFwd | Idler & take up spool out of mesh - adj height of reel with plastic screw |
| Sharp | VCT310HM | Won't accept tape - ok if loaded by hand but stops short on eject | Inner sw on cass housing dirty and check for dries |
| Sharp | VCT510 | No capstan rotation or sound in e-e | sillage on main pcb - replaced motor & ribbon cable and loss of cotinuity to mute tr caused loss of sound |
| Sharp | VCT510HM | Accepts tape but won't fully load - unloads | Drum PG pulses missing - PG coil on drum pcb o/c - resoldered conns to coil restored pulses |
| Sharp | VCT72 | Poor take up torque & tape spills | Check T Up reel soft brake for contamination |
| Sharp | VT3700 | Trips off in any mode but will FF/Rew but not play | Q603 leaky in PSU secondary |
| Sharp | VT3700H | No tuning/sound or accept cassette | IC751 12v reg low - 33v tuning reg IC1005 s/c, R760 220R o/c; D757 15v overvolts diode s/c probably all caused by STRM6523 chopper chip |
| Sharp | VT750 etc | Mechs jams & sw off aft 5 secs | Cam timing keeps jumping out due to broken tooth on supply load pole gear |
| Sharp | VCH81H | No capstan | 10µ 25v cap on motor pcb leaked & corroded track |
| Sharp | VCH92HM | Noisy E-E pic when cold | CQ601 0µ47 50v, CQ602 2µ2 50v in RF/det can |
| Sharp | VCM20HM | Int accepts tape & int stopping | Check end sensors Pt no 0233GEZZ |
| Sharp | VCM301HM | No sound erase | Q651, Q652, R658 4R7 1/4w, C652 0µ1 sm |
| Sharp | VCM301HM | No mech operation - psu ok | Dry joint Q9903 on psu sec |
| Sharp | VCMH60 | Motor pulsing & no display when cold | 5v rail low - C925 470µ 10v, C929 330µ 10v |
| Sharp | VCMH67 | Dead - no 5v | Q961 2SC2001LK |
| Sharp | VCN311 | RF adj | 1/ Put vcr into standby, 2/ Press and hold menu button for 3-5 seconds, 3/ Display will show rf ch, 4/ Use up/down keys to select required rf channel ,5/ Press SET to store. |
| Sharp | VCM302HM | Dead, no display tho psu working | Check for +5V on collector Q703, if not present check/replace Q703,Q704. SMD Q703 B-E s/c could be 2SA1037KQ (marked FQ) or 2PB709AR (marked BR) SMD Q704 o/c could be UN2212 (marked 8B) or DTC124EK (marked 25) They are located near the IR remote sensor. |

| Sharp | VC682SH | No play operation - reel idler not meshing with reel | Mode sw |
| --- | --- | --- | --- |
| Sharp/ Finlandia | VHSLY1 | Slow on pback & rew | brakes perm on - pulley on change over from search to play had ridden down spindle. Fit washer & locking tab on spindle to prevent it |
| Sharp/ Finlandia | VHSSLY | Accepts tape and quickly ejects it | Clean mode sw |
| Sharp/ Granada | VCA40/ VHSHY4 | No mech operation due to no capstan rotation | Change elect cap 10µ on motor pcb & check for corrosion causing o/c print |
| Sharp/ Granada | VHSFY1 | No cahn store | Pin 2 IC5002 should be -31v but was high at -42v. Zener D955 dry joint |