## Descriptions for the operation of factory mode

- AGING MODE

- With factory remote controller to press “D-mode” directly enter AGING mode, screen will display

“factory”. Press “DISP” key to exit “factory” to enter “WHITE BALANCE”.

- With user remote controller to press “MENU” key to turn to “TUNING” menu, press “CH-” key to

Select “TARGET POS” bar, Input“2483”, “6483”, “6568”, then enter AGING mode.

- At “factory” mode or “AGING” mode, press “SYS”, “SOUND”, “SLEEP” key to adjust data, screen

will display relevant menu+factory. Press “MENU” to display main Menu to adjust all data. Press

“DISP” to exit, screen only display “factory”.

- WHITE BALANCE MODE

1) At AGING mode, press “D-mode” exit “factory” to enter WHITE BALANCE mode.

2) Press “D-mode” key two time (factory key) to enter WHITE BALANCE mode.

3) At “factory” mode, press figure key “0” enter WHITE BALANCE mode.

4) At WHITE BALANCE mode, press “MUTE”, one horizontal bright line appear on screen,

use “1,4” key to adjust RCUT, “2,5” key to adjust GCUT, “3,6” key to adjust BCUT

- ENTER FACTORY MODE

- USER REMOTE CONTROLLER

- At first enter AGING mode, at AGING mode, press “DISP” to exit “factory”, enter

WHITE BALANCE mode.

2)  At WHITE BALANCE mode, press figure 1,2,3,4,5,6, DISP to enter “factory” mode.

B.  Press “D-mode” key three time (factory key) to enter factory’s adjustment menu.

- ENTER FACTORY MODE F8~F15

Press “2483”,”6483”,”6568” enter F8, and also open F8-F15. Press “Mute” Key to turn Page Up and

“RETURN” key to turn Page Down for factory mode menu. Press figure Key 1、2、3、4、5、6、7、8、9

to enter “factory” mode  F1~F9。

■  EXIT FACTORY MODE

1）Press “DISP” to exit from “factory” menu.

2)  Press power switch, exit “factory’s mode.

- EXPLANATION FOR ADJUSTMENT SPEED KEY

After entering factory mode, some remote controller key function will change to suit for the production

adjustment.

1)  Press “MUTE” key to turn page up, press “RETURN” key to turn page down.

2)  Press “CH+” and “CH-“ key to select item, and press “VOL+” and “VOL-” key to change data value.

3)  Press “SLEEP” key to display CHIP inner 6 kinds test signal.

0：White Balance adjustment

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| ## RC | R-CUT | 110 | 255 |
| GC | G-CUT | 110 | 255 |
| BC | B-CUT | 110 | 255 |
| GD | G-Diver | 60 | 127 |
| BD | B-Driver | 72 | 127 |
| U．BLK | U signal black level adjust | 8 | 15 |
| V．BLK | V signal black level adjust | 8 | 15 |
| S．RY | SECAM，R signal black level adjust | 8 | 15 |
| S．BY | SECAM，B signal black level adjust | 8 | 15 |

press “MUTE”, one horizontal bright line appear on screen, use “1,4” key to adjust RCUT,

“2,5” key to adjust GCUT, “3,6” key to adjust BCUT

1：Geometry parameter adjustment for PAL (Not available for NTSC)

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| H.PHASE | Horizontal Phase for 50Hz | 18 | ### 31 |
| V.SIZE | Vertical Amplitude for 50Hz | 20 | 63 |
| V.POS | Vertical Phase for 50Hz | 02 | 15 |
| V.LIN | Vertical Linearity for 50Hz | 15 | 31 |
| V.SC | V-S correction for 50Hz | 15 | 31 |
| HBOW | H Bow correction for 50Hz | 4 | ### 7 |
| HPARA | H Para correction for 50Hz | 4 | ### 7 |
| H.BLK.RIGHT | H.Right Blanking | 0 | 07 |
| H.BLK.LEFT | H.Left Blanking | 0 | 07 |

Geometry parameter adjustment for NTSC (offset for PAL system, NTSC only)

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| NT.H.PHASE | Horizontal Phase for 60Hz | 10 | ### 20 |
| NT.V.SISE | Vertical Amplitude for 60Hz | 10 | 20 |
| NT.V.POS | Vertical Phase for 60Hz | 11 | 20 |
| NT.V.LIN | Vertical Linearity for 60Hz | 10 | 20 |
| NT.V.SC | V-S correction for 60Hz | 10 | 31 |
| NT.HBOW | H Bow correction for 60Hz | 4 | ### 7 |
| NT.HPARA | H Para correction for 60Hz | 4 | ### 7 |
| V.BLK.TOP | V.Top Blanking | 0 | 3 |
| V.BLK.BTM | V.Bottom Blanking | 0 | 3 |

2：Brightness、Contrast、Color adjustment

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| SUB.CONT | Sub Contrast | 15 | 15 |
| SUB.BRIGHT | Brightness | 0 | -63~63 |
| SUB.COLOR | Sub Color | 10 | -128~—+127 |
| S.SUB.COLOR | Sub Color for SECAM | 64 | 127 |
| RF.AGC | IF Gain | 30 | 63 |
| ## BACKGROUND | Background 0：OFF 1：ON | 1 | 1 |
| BACK.COLOR | BACK COLOR, 0：blue 1：black | 0 | 1 |
| POW.CONT | Brightness boost speed for “POWER ON” | 5 | 15 |
| LOCK | Child lock | 0 | 1 |
| LOGO MOVE | LOGO MOVE 0：OFF 1：ON | 0 | 1 |

3：OSD1 adjustment

| OSD | Descriptions | Remark | Recommend Data |
| --- | --- | --- | --- |
| MENU BACK | MENU translucent background selection | 0：OFF 1：ON | 1 |
| MENU ICON | MNEU icon display selection | 0：OFF 1：ON | 1 |
| OPEN CURT. | Curtain style when switch on | 0：OFF 1：ON | 1 |
| CLOSE CURT. | Curtain style when switch off | 0：OFF 1： ON | 1 |
| CALENDER | Calendar switch | 0：OFF 1： ON | 1 |
| P.ON.LOGO | LOGO display when power on | 0：OFF 1：SET | 0 |
| NO.SG.LOGO | LOGO display without signal | 0：OFF 1：SET | 0 |
| OSD.CONT | OSD contrast | 0:95IRE1:60IRE2:70IRE3: 80IRE | 1 |
| ANALOG ADJ | Adjust main MENU simulation value, Whether make main MENU disappear | 0：No disappear 1：disappear | 0 |

4：Function control adjustment

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| TV | TV SWITCH 0:OFF 1:ON | 1 | 1 |
| AV2 | AV2 SWITCH 0: OFF 1: ON | 1 | 1 |
| DVD | DVD SWITCH 0: OFF 1: ON | 1 | 1 |
| FM | FM SWITCH 0: OFF 1: ON | 0 | 1 |
| BG | BG 0: OFF 1: ON | 1 | 1 |
| I | I SWITCH 0: OFF 1: ON | 1 | 1 |
| DK | DK SWITCH 0: OFF 1: ON | 1 | 1 |
| M | M SWITCH 0: OFF 1: ON | 1 | 1 |
| FM-BAND | SOUND IF MODULATE WIDTH, 0: BG\DK\I division 1: no division | 0 | 1 |

5: Screen Protection OSD position and H.V. compensation

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| LEFT | Left position limit for Screen protection OSD moving | 16 | 255 |
| RIGHT | Right position limit for Screen protection OSD moving | 125 | 255 |
| TOP | Top position limit for Screen protection OSD moving | 24 | 255 |
| BOTTOM | Bottom position limit for Screen protection OSD moving | 180 | 255 |
| CURT.WAIT.TIME | Wait time for pulling curtain | 10 | 188 |
| CURT.CENT | Center position for pulling curtain | 160 | 255 |
| CURT.STEP | Speed for pulling curtain | 3 | 16 |
| OSD.H.POSI. | Horizontal position for Screen protection OSD | 40 | 127 |
| OSD.V.POSI | Vertical position for Screen protection OSD | 18 | 47 |
| ZOOM | ZOOM | 0 | 15 |

6：OSD2 adjustment

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| POS.TIMER | Black screen time adjustment for exchange channels, 8MS time for data “1” | 32 | 255 |
| CH DARK | Black screen for exchange channels 0:OFF 1:ON | 0 | 1 |
| FRANCE | France 0:OFF 1:ON | 1 | 1 |
| RUSSIAN | Russian 0:OFF 1:ON | 1 | 1 |
| T．VIET | Vietnam 0:OFF 1:ON | 0 | 1 |
| TURKEY | Turkey 0:OFF 1:ON | 0 | 1 |
| ARABIA | Arabia 0:OFF 1:ON | 0 | 1 |
| PERSIA | Persia 0:OFF 1:ON | 0 | 1 |
| GERMAN | German 0:OFF 1:ON | 0 | 1 |
| ITALIAN | Italian 0:OFF 1:ON | 0 | 1 |

7：OSD3 Main MENU Color adjustment

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| UP BOX BACKGROUND | Background color selection for up window of main menu | 15 | 15 |
| UP BOX MOVE | Color selection to pitch on picture in up window of main MENU | 2 | 15 |
| UP ICON BACKGROUND | UP ICON color selection in main MENU | 1 | 07 |
| UP ICON MOVE | Color selection to pitch on UP ICON in main MENU | 4 | 07 |
| DOWN BOX BACKGROUND | Background color selection for down window of main menu | 15 | 15 |
| DOWN BOX MOVE | Color selection to pitch on menu item bar in down window of main MENU | 15 | 15 |
| DOWN TEXT BACKGROUND | Character color selection for down window in main MENU | 2 | 07 |
| DOWN TEXT MOVE | Color selection to pitch on character in down window of main MENU | 4 | 07 |

Press “MENU” key on this page, blue screen MENU appear, and also display “FACTORY”,

pls directly use P+,P-,V+,V- key to adjust all items color in MENU, after adjustment then

press “DISP” to exit.

8：VOLUME Parameters adjustment 1

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| S. AUDIO | When video input, sound input from AV1 or AV2, 0:AV1 1:AV2。 | 0 | 1 |
| AUDIO IN | Sound input combination, 0: Stereo 1: 32PIN 2: 22PIN | 0 | 2 |
| TV.MUTE | MUTE control without TV signal, 0：TV no MUTE 1：MUTE | 1 | 1 |
| AV.MUTE | MUTE control without AV signal, 0：AV no MUTE 1：MUTE | 1 | 1 |
| ALC GAIN | Volume automatic control grade, 0:off ,1：1.1Vp-p, 2：1.6Vp-p, 3：2.3Vp-p | 1 | 3 |
| Sound IC | Sound IC selection, 0: OFF 1: Balance 2:TA1343 3:TA1304 | 0 | 3 |
| S.WOOFER | Woofer switch 0: OFF 1: ON | 0 | 1 |
| BALC | Sound Balance center value | 64 | 114 |
| REMOTE | Remote control code selection: 0:9028-0E0E 1:6122-BF40 2:7461-611C 3:7461-1AA1 4: AUTO | 4 | 4 |

9：High, Middle, Low simulation curve adjustment

| OSD | Descriptions | Minimum | Middle | Recommend Data | Max |
| --- | --- | --- | --- | --- | --- |
| CONTRAST | Contrast | 00 | 95 | 127 | 127 |
| BRIGHT | Brightness | 64 | 69 | 53 | 127 |
| COLOR | Color | 15 | 64 | 69 | 127 |
| TINT | Tint | 40 | 58 | 66 | 127 |
| SHARP | Sharpness | 32 | 32 | X | 63 |
| VOLUME | Volume | 64 | 96 | 127 | 127 |
| BASS | Bass | 24 | 72 | X | 127 |
| TREBLE | Treble | 40 | X | X | 127 |
| WOOFER | Woofer | 114 | X | X | 127 |

10：Chip pins status control

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| ## PANEL KEY DETECT | Whether “POWER ON” to test voltage level for control key PCB, 0： 1：default TOSHIBA | 0 | 1 |
| ## REMOTE KEY LOVE | Whether remote control key has “FAVOURITE” function: 0：as Up,Down,Left,Right key 1：as Favorite, Scan, Music key | 1 | 1 |
| 23PIN | TMPA889X Chip Tint signal input or CVBS input 0：Tint signal 1：CVBS | 0 | 1 |
| 59PIN 50/60HZ-UHF | 0：50/60HZ 1：High level When UHF | 0 | 1 |
| 31PIN | TMPA8891 Chip 31pins function 0：SIF OUT 1: AU OUT 2、3:NO USE | 0 | 2 |
| ## 56PIN 3 STATUS | 56pins function, 3 status ? 0：High & Low two level 1：High, Middle, Low three level | 0 | 1 |
| 56PIN V.CHG | 56pins function,TV/AV1/AV2 three status voltage selection 0：0V 2.5V 5V 1:0V 5V 2.5V | 0 | 1 |
| 64PIN | 64pins function, “POWER ON” level selection: 0:Low level “ON”, 1：High level “ON” | 0 | 1 |
| VIF | mage IF 0：38M 1：38.9M 2：45.75M | 0 | 2 |
| VIDEO DSP USB | AV2 DISPLAY USB | 0 | 1 |

11：Function control

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| ## V.AGC.DC | V. AGC power source selection, 0: external 1: inner | 1 | 1 |
| PIF.LEVER | CVBS output amplitude control, 00:1.1V 3：2.4V | 3 | 3 |
| CVBS.PASS | Whether CVBS output pass band filter, 0: PASS 1: NO PASS | 0 | 1 |
| POS.NO. | Channels quantity selection, 0:248 1:256 | 0 | 1 |
| TUNING.MODE | Tuning mode, 0:VS 1:FS | 0 | 1 |
| TUNER.TYPE | Frequency Synthesize Tuner selection 0:38M 1:38A 2:38.9M 3:38.9A | 0 | 3 |
| ## POWER.TIME | Cold “POWER ON” power pin wait time: 0：0 second, 1：1 second，2：2 seconds，3：3 seconds | 0 | 3 |
| STANBY.MODE | “POWER ON” power pin memory status, 0：Remember status， 1：Stand-by status, remote control to “POWER ON”, 2：Do not remember last time “POWER ON” status, automatic “POWER ON” | 0 | 2 |
| U.BAND | Tuner U band level selection, 0：00 1：11 | 1 | 1 |

12：Register control to ABCL、Brightness

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| ABL.POINT | Start point for automatic brightness control 0:0V,1:-0.2V,2:-0.3V,3:-0.4V | 2 | 3 |
| ACL.ST | Start point for automatic contrast control, 0:0V,1:-0.2V,2:-0.3V,3:-1V(OFF) | 1 | 3 |
| ABL.GAIN | Start point for automatic brightness control gain 0:-0.2V,1:-0.35V,2:-0.5V,3:-0.65V; | 3 | 3 |
| YPL | Brightness peak value, 0:OFF 1:105IRE | 0 | 1 |
| BS.SW | Black level delay switch, 0:OFF 1:105IRE | 1 | 1 |
| BS.START | ## Start point for black level delay 0：30IRE，1：40IRE，2：50IRE，3：75IRE | 3 | 3 |
| ## Y.GAMMA | Brightness Gamma correction 0：OFF，1：78IRE，2：68IRE，3：58IRE | 0 | 3 |
| P.OFF.RGB | “POWER OFF” brightness vanish, 0:OFF 1: Brightness vanish : RGB/Cut as 0 2: Brightness vanish, RGB/Cut as FF | 0 | 2 |
| SHARP.BLANCE | Balance adjustment for Vertical line roll | 4 | 7 |

13：Register control to Color

| OSD | Descriptions | Recommend Data |  |  |  | Max |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TV/PAL | TV/NTSC | AV | DVD |  |
| ## Y.DELAY | Brightness delay | 5 | 3 | 3 | 3 | 7 |
| DEMO.PHASE | Color modulate amplitude & phase | 0 | 1 | 1 | 3 | 2 |
| P/N ID | Color sensitivity | 0 | 0 | 1 | 1 | 1 |
| KILLER.OFF | Brightness Killer switch | 0 | 0 | 0 | 0 | 1 |
| SYS.SHARP | Different status SHARP value | 37 | 37 | 37 | 37 | 63 |

Y  DELAY  TIME

0:0ns   1:40ns  2:80ns  3:120ns  4:160ns  5:200ns  6:240ns  7:280ns

14：Register control to Sound trap

| OSD | Descriptions | Recommend Data |  |  |  | Max |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BG | DK | I | M |  |
| ## S.TRAP F0 | Sound trap center frequency width | 08 | 08 | 08 | 03 | 15 |
| ## S.TRAP Q | Sound trap Q value | 0 | 0 | 0 | 0 | 3 |
| ## S.TRAP GD | Sound trap GD value | 2 | 2 | 2 | 2 | 3 |
| ## S.TRAP HP/LP | Sound trap HP/LP value | 0 | 0 | 0 | 3 | 3 |
| STANDARD | Standard status contrast, brightness, color, sharpness | 50 | 50 | 50 | 50 | 100 |
| MILD | Lild status contrast, brightness, color, sharpness | 30 | 30 | 40 | 30 | 100 |
| VIVID | Vivid status contrast, brightness, color, sharpness | 70 | 70 | 60 | 60 | 100 |
| DYNAMIC | Dynamic status contrast, brightness, color, sharpness | 80 | 80 | 50 | 40 | 100 |

15：H. Synchronization parameters adjustment

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| ## SYNC.DET | H.Sync. test mode, 0: inner sync., 1: external sync. | 0 | 1 |
| SYNC.SEARCH | Inner sync. searching, Sync. signal test mode | 14 | 15 |
| SYNC.BB | Inner sync. blue screen, Sync. signal test mode | 14 | 15 |
| BB.COUNT | Inner sync. Blue screen, Sync. Signal counting quantity | 04 | 255 |
| PYNX | External sync. Status H.Sync. pulse counting maximum | 46 | 63 |
| PYNN | External sync. Status H.Sync. pulse counting minimum | 24 | 63 |
| PYXS | External sync. Searching status H.Sync. pulse counting maximum | 34 | 63 |
| PYNS | External sync. Searching status H.Sync. pulse counting minimum | 30 | 63 |

16：Seldom used adjustment items 1

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| HAFC | AFC gain | 0 | 255 |
| VCEN | V. center value, valid for “+,-” power source | 0 | 3F |
| NOISE.SW | Noise test switch 0:OFF 1:ON | 0 | 1 |
| NOISE.COUNT | Test and confirm the noise counting value | 31 | 255 |
| SEARCH1 | Automatic searching calculation 1, 0：quick 1：slow | 0 | 1 |
| SEARCH2 | Automatic searching calculation 2, 0：quick 1：slow | 0 | 1 |
| NOISE | Test the noise comparison value | F | 15 |
| UCOM | Select input mode for test pattern from U-COM | 0 | 7 |
| CH SHADE | Fade in when change 0:OFF 1:ON | 0 | 1 |
| DSP POSITION | TV Station display position 0：right 1：left | 0 | 1 |

17：Seldom used adjustment items 2

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| HVBSW | H/V Blanking on/OFF switch, 0：OFF 1：ON | 0 | 1 |
| C.TRP.Q | C-Trap Q, 0:low 1:high | 0 | 1 |
| AFTCH | AFT check, 0:check AFT for TV or AV, 1: only check AFT for TV | 1 | 1 |
| SEAUD | Automatic tuning，whether search signal downwards 0：search downwards 1：no | 0 | 1 |
| BYRGB | Black screen switch Y/RGB option 0：off Y 1：off RGB | 0 | 1 |
| UV.SW | UV SW 0:Cb/Cr 1:U/V | 0 | 1 |
| C.TRAP | C trap mode selection, 00:01 interlocking “video sw” 10 ：off 11：on | 0 | 3 |
| YUV.DIS | YUV display status 0：YUV 1：DVB | 0 | 1 |
| TINTP | TINT polarity 0: Plus 1：Minus | 1 | 1 |

18：Seldom used adjustment items 3

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| VFREQ | V.frequency setting without signal, 0：automatic, 1：313 | 1 | 1 |
| OVMO | Over modulation switch 0：normal set 1：ON | 0 | 1 |
| AFT SENS. | AFT sensatory 0：wide(250hz )1:normal(83hz) | 0 | 1 |
| NYBUSW | Nyquist Buzz reducer Switch, 0：ON，1： OFF | 1 | 1 |
| SHARPF0 | Select sharpness center frequency, 0：2.75MHz， 1：4MHz | 0 | 1 |
| VCOADJ | Whether adjust VC0 when change channel, 0：adjust VCO ，1：no | 0 | 1 |
| HSTOP | power on H STOP 0:H STOP=0，1:H STOP=1 | 0 | 1 |
| SYNC.SEP. | H. Sync. division degree, 0：40% 1：50% | 1 | 1 |
| SYNC.SLI. | Weak signal sync. Level, 0：standard 1：low | 0 | 1 |
| SYNC.SKEW | Test H.Sync. incline mode, 0：30% 1：50% | 0 | 1 |

19：SECAM adjustment items

| OSD | Descriptions | Recommend Data | Max |
| --- | --- | --- | --- |
| SECAM.GP | 00:normal 01:+200ns 10:center 11:-200ns | 0 | 1 |
| SECAM.INT | secam inhibit swich 0:detecting 1:inhibit | 0 | 1 |
| SECAM.ID | SECAM sensatory 0:normal 1:low | 1 | 1 |
| SECAM.BEL.SW | sel secam bell filter bandwith 0: bell filter 1:boost mode | 0 | 1 |
| SECAM.ID.MOD | select secam ident mode 0:H ID 1:H+V ID | 0 | 1 |
| SECAM.BLACK.SW | select YS-SW mode 0:normal operation 1:secam black level alignment mode | 0 | 1 |
| SECAM.BELL | select secam bell filter Q 0:low 1:high | 1 | 1 |

LOGO seting：At AV mode，press remote figure key “2 4 8 3”、“6483”、“6568” to enter LOGO setting

MENU, according to MENU to select item, press P+、P-、V+、V- key to set LOGO

Parameters, after setting, press DISP key to exit