TV: Service mode remote control codes

| Revision history |  |
| --- | --- |
| 2003-04-12 | •; Removed SERVICE PROFESSIONAL TOOL remote which is replaced by the SERVICE TOOL remote. |
| 2003-04-08 | •; Updated information on the SERVICE TOOL remote (this remote control was kindly provided by Perfect Electronica). |
| 2003-04-06 | •; Changed information on the SERVICE PROFESSIONAL remote control (link and photo). |
| 2003-01-04 | •; Added information on the SERVICE PROFESSIONAL remote control. |
| 2002-10-24 | •; Added sample Pronto sequences. |
| 2002-06-28 | •; Added comments on sequences. |
| 2002-04-14 | •; Added GFL Escape sequence. |

This pages shows, some, remote control codes which are used to access service mode for Philips TV's. If you are lucky you have succeeded in accessing service mode using the codes mentioned on the Matchline service mode page. If you did not succeed (your TV is probably an older type/software), then there is an alternative method.

The usual way of accessing service mode is using a (special) remote control (RC7150) also known as a Dealer Service Tool (DST). If you don't own such a special remote then you might be able to use a programmable remote control.


Building your own programmable remote control
If you don't have a programmable remote then you might be interested into building it yourself. The Projects: Remote control makes it possible. It contains very little components and is very easy to build. It also contains a basic explanation on how to generate the service mode codes using  such a home-built control.


The SERVICE TOOL remote control
This remote control (obtainable from Perfect Electronica at  www.perfect.nu) for about Euro 50,- can be used to activate the service mode for most Philips, Erres, Aristona and many Orion TV's. This following sets are supported: Institutional Anubis B (Hotel TV), L6, L7, MD1.2, MD2., MD2.2. and GFL TV sets.
See one of the tables below for a picture and the codes generated by this remote. This remote control is the successor of the SERVICE PROFESSIONAL remote control which has less keys but has otherwise the same functionality.


Pronto
Pronto owners (the Pronto is a programmable remote control) have the means of entering these remote control codes directly. However, this will not always work. The reason for this is, probably, timing issues. These remote control codes must follow each other within a certain window of time (maximum delay) to be recognized as valid codes.

A real world example:
TV model: PW9525
Remote control codes sequence: RC5 10 26, RC5 16 7, RC5 21 92, RC5 31 4 
This should present the user with a password access screen (but it does not!). However, if we create a 'learned code' sequence of these codes (with some care) then the TV will respond.
To create such a 'learned code' sequence you can use the < RC5PRONTO (206 kb) program. This program generates 'learned' code of multiple RC5 codes with a definable delay inbetween them. 


Remote control list
This list gives the remote control sequences which will get you into service mode. You should use the appropiate sequence, according to your TV (chassis) model.
Note that the delays are for reference only and can be different for certain TV models. The delay is the necessary time required before the next code is to be send. The last delay is usually not necessary. Also note that the codes within a sequence must follow each other within a certain maximum time to be recognized as valid codes. This is why it is most likely not possible to enter the sequences manually (key-by-key).

Each entry in the code sequence is of the following form:

|  | RC5 ss cc |
| --- | --- |

where

|  | ss = system address |
| --- | --- |

|  | cc = command |
| --- | --- |

Note: if you get into service mode you can back up to the parent menu using the MENU button. Alternatively you can use the red teletext button to get the same result (thanks to Eddie Talbot).

| Description | Code sequence | Delay(ms) | Comment | Test comments | Generated sequence (RC5PRONTO) samples (untested unless otherwise indicated!) |
| --- | --- | --- | --- | --- | --- |
| GFL Alignment i.e MG3. | RC5 10 26 | 25 |  | Tested with a PW9525 and Pronto. 'Learn' only first 4 codes with a 25 ms delay setting. You get "Service Entry, Enter password and press OK". Use 3140 as password.. Or (as a single definition):. [C] LEARNED (first 4 codes only). [D] 1.0 (delay). [C] RC5 0 3. [D] 0.3. [C] RC5 0 1. [D] 0.3. [C] RC5 0 4. [D] 0.3. [C] RC5 0 0. [D] 0.3. [C] RC5 0 87. | 0000 0070 0068 0000 0021 0021 0042 0021 0021 0042 0042 0042 0042 0021 0021 0042 0021 0021 0042 0042 0042 039D 0021 0021 0042 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 039D 0042 0021 0021 0042 0042 0042 0042 0042 0042 0042 0021 0021 0021 0021 0042 0021 0021 4849 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0042 0042 0021 0021 4849 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 1CEA 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 1CEA 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 1CEA 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 1CEA 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0042 0021 0021 0021 0021 0021 2B5F TESTED |
|  | RC5 16 7 | 25 |  |  |  |
|  | RC5 21 92 | 500 | 25 ms works too |  |  |
|  | RC5 31 4 | 500 |  |  |  |
|  | RC5 0 3 | 200 | These codes can be entered 'manually', they have no timing restrictions and are normal codes, namely: 3140 OK. |  |  |
|  | RC5 0 1 | 200 |  |  |  |
|  | RC5 0 4 | 200 |  |  |  |
|  | RC5 0 0 | 200 |  |  |  |
|  | RC5 0 87 | 300 |  |  |  |
| GFL Service default | RC5 10 26 | 25 |  | Tested with a PW9525 and Pronto. | 0000 0070 002A 0000 0021 0021 0042 0021 0021 0042 0042 0042 0042 0021 0021 0042 0021 0021 0042 0042 0042 039D 0021 0021 0042 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 039D 0042 0021 0021 0042 0042 0042 0042 0042 0042 0042 0021 0021 0021 0021 0042 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 2B5F TESTED |
|  | RC5 16 7 | 25 |  |  |  |
|  | RC5 21 92 | 1000 | 25 ms works too |  |  |
|  | RC5 31 3 | 300 |  |  |  |
| GFL Dealer options | RC5 10 26 | 25 |  | Tested with a PW9525 and Pronto. | 0000 0070 0029 0000 0021 0021 0042 0021 0021 0042 0042 0042 0042 0021 0021 0042 0021 0021 0042 0042 0042 039D 0021 0021 0042 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 039D 0042 0021 0021 0042 0042 0042 0042 0042 0042 0042 0021 0021 0021 0021 0042 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0042 2B5F TESTED |
|  | RC5 16 7 | 25 |  |  |  |
|  | RC5 21 92 | 1000 | 25 ms works too |  |  |
|  | RC5 31 2 | 300 |  |  |  |
| GFL Reset/Escape | RC5 10 26 | 25 |  | This acts as a kind of reset/escape if you are stuck. An invalid GFL command will display a 'Dealer' message with no way of escaping other than a valid command or this one. This command also works for the regular user menu's.. | 0000 0070 002A 0000 0021 0021 0042 0021 0021 0042 0042 0042 0042 0021 0021 0042 0021 0021 0042 0042 0042 039D 0021 0021 0042 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 039D 0042 0021 0021 0042 0042 0042 0042 0042 0042 0042 0021 0021 0021 0021 0042 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 039D TESTED |
|  | RC5 16 7 | 25 |  |  |  |
|  | RC5 21 92 | 25 |  |  |  |
|  | RC5 31 0 | 25 |  |  |  |
| GFL Reset/Escape | RC5 31 0 |  |  | Can also be used in some cases to escape. | 0000 0070 000C 0000 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 039D |
| GR2.X Open factory - Service | RC5 0 22 | 25 | Factory open | Tested with a ST2781 (GR2.2) and Pronto. If you omit the (2) marked codes you get into FAC mode.. Using the (2) marked codes with or without the (1) marked codes get you into SER FAC mode.. See TV: MISCELLANEOUS (GR2.2). | 0000 0070 0052 0000 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0042 0021 0021 0042 039D 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0042 0021 0021 0021 0021 0021 039D 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0042 0021 0021 0021 0021 2B5F 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 039D 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0042 0042 0021 0021 0042 0021 0E75 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 039D 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0042 0021 0021 0042 0021 2B5F |
|  | RC5 0 23 | 25 |  |  |  |
|  | RC5 0 24 | 300 |  |  |  |
|  | RC5 3 3 | 25 | (1) Open service version 1 |  |  |
|  | RC5 0 41 | 100 | (1) |  |  |
|  | RC5 3 0 | 25 | (2) Open service version 2 |  |  |
|  | RC5 0 89 | 300 | (2) |  |  |
| GR2.X White Peak Limit + | RC5 0 116 | 100 |  | Not tested | 0000 0070 000B 0000 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0042 0042 0042 0021 0021 0E75 |
| FL1 (old) Open factory - alignment | RC5 28 30 | 25 | Factory open | Not tested | 0000 0070 0037 0000 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 039D 0021 0021 0042 0042 0021 0021 0021 0021 0042 0042 0021 0021 0042 0042 0042 0021 0021 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0042 0021 0021 0042 0042 9092 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0021 0021 0042 0042 56BE 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 2B5F |
|  | RC5 29 40 | 25 |  |  |  |
|  | RC5 30 50 | 1000 |  |  |  |
|  | RC5 3 50 | 600 | Alignment open |  |  |
|  | RC5 31 63 | 300 | Factory close |  |  |
| FL1 (old) Store/Close factory - alignment | RC5 28 30 | 25 | Factory open | Not tested | 0000 0070 0042 0000 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 039D 0021 0021 0042 0042 0021 0021 0021 0021 0042 0042 0021 0021 0042 0042 0042 0021 0021 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0042 0021 0021 0042 0042 9092 0042 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0042 0042 0021 0021 0042 0042 0042 56BE 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0021 0021 0042 0021 0021 0021 56BE 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 2B5F |
|  | RC5 29 40 | 25 |  |  |  |
|  | RC5 30 50 | 1000 |  |  |  |
|  | RC5 3 90 | 600 |  |  |  |
|  | RC5 3 51 | 600 |  |  |  |
|  | RC5 31 63 | 300 | Factory close |  |  |
| FL1 Open factory - alignment | RC5 28 35 | 25 | Factory open | Tested with a PW9525 and Pronto. Note that you can not change dealer options -> if you select the dealer options and then exit the menu some 'residual' text will be left on screen.. | 0000 0070 0045 0000 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0042 0042 0042 0042 0021 0021 0042 0042 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0021 0021 0021 9092 0042 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0021 0021 9092 0042 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 9092 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 2B5F TESTED |
|  | RC5 30 45 | 25 |  |  |  |
|  | RC5 31 55 | 1000 |  |  |  |
|  | RC5 16 72 | 1000 | Alignment open version 1 (you can omit this for the PW9525). |  |  |
|  | RC5 16 64 | 1000 | Alignment open version ptv |  |  |
|  | RC5 31 63 | 300 | Factory close |  |  |
| FL1 Open factory - white screen | RC5 28 35 | 25 | Factory open | Seems to work with the PW9525 (the screen lights up white momentarily if you use the whole sequence). If you omit the 'factory close' code then you will have a completely white screen. Sending the 'factory close' will end it.. | 0000 0070 0044 0000 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0042 0042 0042 0042 0021 0021 0042 0042 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0021 0021 0021 9092 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 039D 0021 0021 0042 0021 0021 0042 0021 0021 0042 0042 0021 0021 0042 0042 0042 0021 0021 0021 0021 9092 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 2B5F TESTED |
|  | RC5 30 45 | 25 |  |  |  |
|  | RC5 31 55 | 1000 |  |  |  |
|  | RC5 2 1 | 25 | White |  |  |
|  | RC5 13 40 | 1000 |  |  |  |
|  | RC5 31 63 | 300 | Factory close |  |  |
| FL1 Open factory - red screen | RC5 28 35 | 25 | Factory open | Seems to work with the PW9525 (the screen lights up red momentarily if you use the whole sequence). If you omit the 'factory close' code then you will have a completely red screen. Sending the 'factory close' will end it.. | 0000 0070 0043 0000 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0042 0042 0042 0042 0021 0021 0042 0042 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0021 0021 0021 9092 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 039D 0021 0021 0042 0021 0021 0042 0021 0021 0042 0042 0021 0021 0042 0042 0042 0021 0021 0021 0021 9092 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 2B5F TESTED |
|  | RC5 30 45 | 25 |  |  |  |
|  | RC5 31 55 | 1000 |  |  |  |
|  | RC5 2 2 | 25 | Red |  |  |
|  | RC5 13 40 | 1000 |  |  |  |
|  | RC5 31 63 | 300 | Factory close |  |  |
| FL1 Open factory - green screen | RC5 28 35 | 25 | Factory open | Seems to work with the PW9525 (the screen lights up green momentarily if you use the whole sequence). If you omit the 'factory close' code then you will have a completely green screen. Sending the 'factory close' will end it.. | 0000 0070 0044 0000 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0042 0042 0042 0042 0021 0021 0042 0042 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0021 0021 0021 9092 0021 0021 0042 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 039D 0021 0021 0042 0021 0021 0042 0021 0021 0042 0042 0021 0021 0042 0042 0042 0021 0021 0021 0021 9092 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 2B5F TESTED |
|  | RC5 30 45 | 25 |  |  |  |
|  | RC5 31 55 | 1000 |  |  |  |
|  | RC5 2 3 | 25 | Green |  |  |
|  | RC5 13 40 | 1000 |  |  |  |
|  | RC5 31 63 | 300 | Factory close |  |  |
| FL1 Open factory - default (error list) | RC5 28 35 | 25 | Factory open | Tested with a PW9525 and Pronto. Note: STANDBY will not exit service default mode: use the GFL service default and then put in standby to exit it. | 0000 0070 003A 0000 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0042 0042 0021 0021 0021 0021 0042 0021 0021 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0042 0042 0042 0042 0021 0021 0042 0042 0021 039D 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0042 0042 0021 0021 0021 0021 0021 9092 0021 0021 0042 0042 0042 0021 0021 0021 0021 0021 0021 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 9092 0021 0021 0042 0042 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 0021 2B5F |
|  | RC5 30 45 | 25 |  |  |  |
|  | RC5 31 55 | 1000 |  |  |  |
|  | RC5 16 63 | 1000 | Default |  |  |
|  | RC5 31 63 | 300 | Factory close |  |  |

Additional sequences which work with the PW9525 model (and probably works for all MG2.1/MG3.1/MG4.1/MG7.1 related models):

| Description | Code sequence | Delay(ms) | Comment | Test comments |
| --- | --- | --- | --- | --- |
| PW9525 Lock remote | RC5 28 35 | 25 | This 'locks' the TV so it will not accept any remote control codes. You need to turn the TV off to reverse it. The mechanical controls on the TV itself keep functioning.. The reason is simple: since this is part of a FL1 sequence it expects more codes.... | Tested with a PW9525 and Pronto. |
|  | RC5 30 45 | 25 |  |  |
|  | RC5 31 55 | 25 |  |  |

|  | SERVICE TOOL remote control |  |  |  |
| --- | --- | --- | --- | --- |
|  | This remote control was kindly provided by Perfect Electronica (www.perfect.nu). Successor of the SERVICE PROFESSIONAL remote control (same functionality).. Besides some minor differences in delays the only difference with the GFL sequence code table above, is the repetition of the last command in some cases.. |
|  | Description/key | Code sequence | Delay(ms) | Comment |
|  | INSTALL | RC5 0 122 |  | Normal customer installation. |
|  | STANDBY | RC5 0 12 |  |  |
|  | 0-9 | RC5 0 x |  | 'x' ranges from 0-9 |
|  | OSD | RC5 0 15 |  | Display |
|  | SLEEP | RC5 0 38 |  | Sleep timer |
|  | I-II | RC5 0 35 |  | Language 1/2 |
|  | MUTE | RC5 0 13 |  |  |
|  | A/CH | RC5 0 34 |  | Alternate channel |
|  | UP | RC5 0 80 |  |  |
|  | PICT | RC5 3 10 |  |  |
|  | LEFT | RC5 0 85 |  |  |
|  | OK | RC5 0 87 |  | Acknowledge |
|  | SOUND | RC5 3 11 |  |  |
|  | INCR.SOUND | RC5 0 36 |  | Spatial stereo |
|  | DOWN | RC5 0 81 |  |  |
|  | AV | RC5 0 56 |  | External |
|  | PROG+ | RC5 0 32 |  |  |
|  | VOL- | RC5 0 17 |  |  |
|  | M | RC5 0 87 |  |  |
|  | VOL+ | RC5 0 16 |  |  |
|  | PROG- | RC5 0 33 |  |  |
|  | A - RED - PICT | RC5 0 107 |  |  |
|  | B - GREEN - SND | RC5 0 108 |  |  |
|  | C - YELLOW - FEAT | RC5 0 109 |  |  |
|  | D - BLUE | RC5 0 110 |  |  |
|  | E - WHITE | RC5 0 111 |  | Index |
|  | TELETEXT | RC5 0 60 |  |  |
|  | SIZE | RC5 0 41 |  | Store/execute/vote |
|  | ? | RC5 0 44 |  | Decrement |
|  | X | RC5 0 45 |  |  |
|  | PICTURE FORMAT | RC5 0 126 |  |  |
|  | ALIGN | RC5 10 26 | 15 | Use '3140' as password when asked for. |
|  |  | RC5 16 7 | 20 |  |
|  |  | RC5 21 92 | 225 |  |
|  |  | RC5 31 4 | 5 |  |
|  |  | RC5 31 4 |  |  |
|  | DEFAULT | RC5 10 26 | 15 |  |
|  |  | RC5 16 7 | 20 |  |
|  |  | RC5 21 92 | 225 |  |
|  |  | RC5 31 3 | 5 |  |
|  |  | RC5 31 3 |  |  |
|  | DEALER | RC5 10 26 | 15 |  |
|  |  | RC5 16 7 | 20 |  |
|  |  | RC5 21 92 | 225 |  |
|  |  | RC5 31 2 | 5 |  |
|  |  | RC5 31 2 |  |  |
|  | EXIT | RC5 31 0 |  |  |
|  | TV | RC5 0 63 |  |  |