Windows EDID V1.0 User Manual

Operating System: MS Windows 98, 2000, XP

Port Setup: Windows 98  Don’t need setup

Windows 2000, XP  Need to Port Setup.

This program is available to LCD Monitor only.

1. Port Setup

- Copy “UserPort.sys” file to “c:\WINNT\system32\drivers” folder

- Run Userport.exe

- Remove all default number

- Add 300-3FF

e) Click Start button.

f) Click Exit button.

2. EDID Read & Write

1) Run WinEDID.exe

2) Edit Week of Manufacture, Year of Manufacture, Serial Number

- Input User Info Data

- Click “Update” button

- Click “ Write” button